IT → OT DMZ → OT Cross-Zone Use Case Library

Sigma Detection Rules Mapped to the Purdue Model (L5 through L0)

Prepared for: Mahesh — OT/ICS SOC, Help AG

Environment: Windows/AD, Firewall/Zone Enforcement, Nozomi Guardian/Vantage, Zeek + ICSNPP

Date: 1 July 2026

Purpose

This library covers the boundary-crossing detection use cases that complement the protocol-level OT/ICS Sigma library (Modbus, DNP3, OPC UA, S7comm, IEC 104). Most real-world OT-impacting incidents follow exactly this path: initial access in IT, lateral movement through the OT DMZ, and final impact at the supervisory or control layer. Rules are grouped by the Purdue Model boundary they monitor, from the IT/Enterprise zone (L4/L5) down to field devices (L0).

Field names, zone labels (IT, OT_DMZ, L3, L2_HMI, etc.), and reference lookup lists (jump-host inventories, HMI-to-PLC pairing maps, engineering role groups) are placeholders and must be populated from the site’s actual network segmentation design, asset inventory, and Active Directory group membership before deployment.

# Table of Contents

# Use Case Summary

| # | Title | Zone Boundary | ATT&CK | Level |
| --- | --- | --- | --- | --- |
| 1 | Direct IT-to-OT-DMZ Connection Bypassing Jump Host | IT (L4/L5) → OT DMZ (L3.5) | attack.ics.t0886, attack.t1021 | HIGH |
| 2 | Unauthorized Lateral-Movement Protocol Entering OT DMZ | IT (L4/L5) → OT DMZ (L3.5) | attack.ics.t0866, attack.t1021.002, attack.t1021.006 | HIGH |
| 3 | OT DMZ Historian Replication Direction Anomaly | IT (L4/L5) → OT DMZ (L3.5) | attack.ics.t0886, attack.ics.t0840 | HIGH |
| 4 | AV/Patch Management Push to DMZ Outside Maintenance Window or Scope | IT (L4/L5) → OT DMZ (L3.5) | attack.t1072, attack.ics.t0843 | MEDIUM |
| 5 | OT DMZ Asset External DNS or NTP Resolution Attempt | IT (L4/L5) → OT DMZ (L3.5) | attack.t1071.004, attack.t1568 | MEDIUM |
| 6 | Web or Email Content Landing on OT DMZ Jump Host | IT (L4/L5) → OT DMZ (L3.5) | attack.t1566, attack.ics.t0817 | CRITICAL |
| 7 | Jump Host Issuing OT Control Protocol Traffic Directly | OT DMZ (L3.5) → Site Operations (L3) | attack.ics.t0867, attack.ics.t0855 | CRITICAL |
| 8 | Remote Access Session into L3 Without Approved Change Window | OT DMZ (L3.5) → Site Operations (L3) | attack.t1133, attack.ics.t0859 | MEDIUM |
| 9 | Shared or Service Account Used From Unusual DMZ Source | OT DMZ (L3.5) → Site Operations (L3) | attack.t1078, attack.ics.t0859 | HIGH |
| 10 | File Transfer From DMZ to L3 With Executable or Engineering Project Extension | OT DMZ (L3.5) → Site Operations (L3) | attack.ics.t0843, attack.t1105 | HIGH |
| 11 | Vendor Remote Access Session Duration or Scope Anomaly | OT DMZ (L3.5) → Site Operations (L3) | attack.t1133, attack.ics.t0883 | MEDIUM |
| 12 | Unscheduled HMI Project or Configuration Deployment | Site Operations (L3) → Supervisory (L2 — HMI/SCADA) | attack.ics.t0831, attack.ics.t0843 | MEDIUM |
| 13 | New or Modified Scheduled Task on HMI or SCADA Server | Site Operations (L3) → Supervisory (L2 — HMI/SCADA) | attack.t1053.005, attack.ics.t0839 | HIGH |
| 14 | Engineering Software Launched by Non-Engineering Account | Site Operations (L3) → Supervisory (L2 — HMI/SCADA) | attack.ics.t0858, attack.t1078 | HIGH |
| 15 | Abnormal Authentication Pattern on SCADA or HMI Server | Site Operations (L3) → Supervisory (L2 — HMI/SCADA) | attack.t1110, attack.ics.t0859 | MEDIUM |
| 16 | Unexpected Outbound Connection from L2 HMI Toward DMZ or IT | Site Operations (L3) → Supervisory (L2 — HMI/SCADA) | attack.ics.t0886, attack.t1041 | HIGH |
| 17 | Control Command From HMI Outside Documented HMI-to-PLC Pairing | Supervisory (L2) → Control (L1 — PLC/RTU) | attack.ics.t0855, attack.ics.t0867 | HIGH |
| 18 | PLC Logic or Firmware Upload From Non-Engineering Host | Supervisory (L2) → Control (L1 — PLC/RTU) | attack.ics.t0843, attack.ics.t0857 | CRITICAL |
| 19 | Engineering Protocol Session Opened by HMI Account | Supervisory (L2) → Control (L1 — PLC/RTU) | attack.ics.t0867, attack.ics.t0858 | HIGH |
| 20 | Safety Instrumented System Point Forced or Overridden | Control (L1) → Field Devices (L0) | attack.ics.t0837, attack.ics.t0800 | CRITICAL |
| 21 | Setpoint Change Exceeding Safe Delta Threshold | Control (L1) → Field Devices (L0) | attack.ics.t0836, attack.ics.t0855 | HIGH |
| 22 | Redundant Sensor Correlation Divergence | Control (L1) → Field Devices (L0) | attack.ics.t0856, attack.ics.t0804 | MEDIUM |
| 23 | First-Time Asset Communication Across a Zone Boundary | Cross-Cutting (All Zone Boundaries) | attack.ics.t0883, attack.ics.t0846 | MEDIUM |
| 24 | Zone Enforcement Firewall Rule Hit-Count Anomaly | Cross-Cutting (All Zone Boundaries) | attack.ics.t0886 | MEDIUM |
| 25 | Removable Media Event on DMZ or L3 Asset | Cross-Cutting (All Zone Boundaries) | attack.ics.t0847, attack.t1091 | HIGH |
| 26 | East-West Traffic Between L1 Controllers Across Process Cells | Cross-Cutting (All Zone Boundaries) | attack.ics.t0867, attack.ics.t0883 | HIGH |
| 27 | NTP or Time Desynchronization Anomaly Across IT-OT Path | Cross-Cutting (All Zone Boundaries) | attack.ics.t0851, attack.ics.t0804 | LOW |

# IT (L4/L5) → OT DMZ (L3.5)

## Direct IT-to-OT-DMZ Connection Bypassing Jump Host

Detects a network flow originating from an enterprise IT subnet landing directly on an OT DMZ host without traversing the designated jump server / reverse proxy, indicating a possible firewall misconfiguration or an attacker pivoting around the documented access path.

| Rule ID | d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e01 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0886, attack.t1021 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Direct IT-to-OT-DMZ Connection Bypassing Jump Host id: d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e01 status: experimental description: Detects a network flow originating from an enterprise IT subnet landing directly on an OT DMZ host without traversing the designated jump server / reverse proxy, indicating a possible firewall misconfiguration or an attacker pivoting around the documented access path. references: - https://attack.mitre.org/techniques/T0886/ tags: - attack.ics.t0886 - attack.t1021 author: Mahesh date: 2026/07/01 logsource: category: firewall product: generic detection: selection: src_zone: IT dst_zone: OT_DMZ filter: dst_ip¦in_list: 'approved_jump_host_ips' condition: selection and not filter falsepositives: - Newly deployed DMZ service not yet added to the jump-host allow-list level: high

## Unauthorized Lateral-Movement Protocol Entering OT DMZ

Detects RDP, SMB, PsExec/Admin-share, or WinRM traffic originating from corporate IT IP ranges and terminating on OT DMZ hosts, consistent with lateral movement techniques used to pivot from an IT compromise toward OT (as seen in NotPetya/TRITON-style intrusion paths).

| Rule ID | d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e02 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0866, attack.t1021.002, attack.t1021.006 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Unauthorized Lateral-Movement Protocol Entering OT DMZ id: d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e02 status: experimental description: Detects RDP, SMB, PsExec/Admin-share, or WinRM traffic originating from corporate IT IP ranges and terminating on OT DMZ hosts, consistent with lateral movement techniques used to pivot from an IT compromise toward OT (as seen in NotPetya/TRITON-style intrusion paths). references: - https://attack.mitre.org/techniques/T0866/ - https://attack.mitre.org/techniques/T1021/ tags: - attack.ics.t0866 - attack.t1021.002 - attack.t1021.006 author: Mahesh date: 2026/07/01 logsource: category: firewall product: generic detection: selection: src_zone: IT dst_zone: OT_DMZ dst_port: - 3389 - 445 - 5985 - 5986 condition: selection falsepositives: - Authorized IT administrator performing approved DMZ server patching via documented break-glass account level: high

## OT DMZ Historian Replication Direction Anomaly

Detects a DMZ historian/replication host initiating a connection back into the IT/Enterprise zone, when the documented data-flow design is one-directional (OT to DMZ to IT) via a data diode or unidirectional gateway.

| Rule ID | d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e03 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0886, attack.ics.t0840 |
| Severity | HIGH |

Sigma Rule (YAML):

title: OT DMZ Historian Replication Direction Anomaly id: d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e03 status: experimental description: Detects a DMZ historian/replication host initiating a connection back into the IT/Enterprise zone, when the documented data-flow design is one-directional (OT to DMZ to IT) via a data diode or unidirectional gateway. references: - https://attack.mitre.org/techniques/T0886/ tags: - attack.ics.t0886 - attack.ics.t0840 author: Mahesh date: 2026/07/01 logsource: category: firewall product: generic detection: selection: src_zone: OT_DMZ dst_zone: IT src_ip¦in_list: 'dmz_historian_ips' filter: timeframe_minute¦range: [0, 5] # replace with actual scheduled replication window check condition: selection and not filter falsepositives: - Misconfigured replication schedule causing connections outside the documented window level: high

## AV/Patch Management Push to DMZ Outside Maintenance Window or Scope

Detects an enterprise WSUS/SCCM/antivirus management console pushing content to OT DMZ hosts outside the approved patch window, or to a DMZ host not listed in its managed device scope, which can indicate abuse of a trusted management channel for malware staging.

| Rule ID | d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e04 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.t1072, attack.ics.t0843 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: AV/Patch Management Push to DMZ Outside Maintenance Window or Scope id: d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e04 status: experimental description: Detects an enterprise WSUS/SCCM/antivirus management console pushing content to OT DMZ hosts outside the approved patch window, or to a DMZ host not listed in its managed device scope, which can indicate abuse of a trusted management channel for malware staging. references: - https://attack.mitre.org/techniques/T1072/ tags: - attack.t1072 - attack.ics.t0843 author: Mahesh date: 2026/07/01 logsource: category: process_creation product: windows detection: selection: Image¦endswith: - '\ccmexec.exe' - '\wuauclt.exe' filter_host: ComputerName¦in_list: 'dmz_managed_hosts' filter_time: hour¦gte: 1 hour¦lte: 4 condition: selection and not (filter_host and filter_time) falsepositives: - Emergency out-of-band patch deployment approved via change ticket level: medium

## OT DMZ Asset External DNS or NTP Resolution Attempt

Detects a DMZ host attempting DNS or NTP resolution against a server outside the small, fixed internal set normally permitted, which can indicate C2 beaconing, DNS tunneling, or an attempt to set up time-based evasion.

| Rule ID | d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e05 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.t1071.004, attack.t1568 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: OT DMZ Asset External DNS or NTP Resolution Attempt id: d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e05 status: experimental description: Detects a DMZ host attempting DNS or NTP resolution against a server outside the small, fixed internal set normally permitted, which can indicate C2 beaconing, DNS tunneling, or an attempt to set up time-based evasion. references: - https://attack.mitre.org/techniques/T1071/004/ tags: - attack.t1071.004 - attack.t1568 author: Mahesh date: 2026/07/01 logsource: category: dns product: generic detection: selection: src_zone: OT_DMZ filter: dst_ip¦in_list: 'approved_internal_dns_ntp_servers' condition: selection and not filter falsepositives: - Newly added internal DNS/NTP server not yet in the allow-list level: medium

## Web or Email Content Landing on OT DMZ Jump Host

Detects browser or email-client process execution, or a file with a web-download/email-attachment zone-identifier marker, on a DMZ jump host. Jump hosts should be hardened bastion systems with no internet browsing or email capability; any such activity is high-confidence compromise or severe policy violation.

| Rule ID | d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e06 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.t1566, attack.ics.t0817 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: Web or Email Content Landing on OT DMZ Jump Host id: d1a1b2c3-0001-4a1b-8c1d-1a2b3c4d5e06 status: experimental description: Detects browser or email-client process execution, or a file with a web-download/email-attachment zone-identifier marker, on a DMZ jump host. Jump hosts should be hardened bastion systems with no internet browsing or email capability; any such activity is high-confidence compromise or severe policy violation. references: - https://attack.mitre.org/techniques/T1566/ tags: - attack.t1566 - attack.ics.t0817 author: Mahesh date: 2026/07/01 logsource: category: process_creation product: windows detection: selection: ComputerName¦in_list: 'dmz_jump_hosts' Image¦endswith: - '\outlook.exe' - '\chrome.exe' - '\msedge.exe' - '\firefox.exe' condition: selection falsepositives: - None expected on a properly hardened jump host level: critical

# OT DMZ (L3.5) → Site Operations (L3)

## Jump Host Issuing OT Control Protocol Traffic Directly

Detects a designated RDP/jump-server host appearing as a Modbus, DNP3, S7comm, or OPC UA protocol client itself rather than simply brokering an interactive remote session, which can indicate an attacker has installed tooling directly on the jump host to issue OT commands.

| Rule ID | d1a1b2c3-0002-4a1b-8c1d-2a2b3c4d5e01 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0867, attack.ics.t0855 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: Jump Host Issuing OT Control Protocol Traffic Directly id: d1a1b2c3-0002-4a1b-8c1d-2a2b3c4d5e01 status: experimental description: Detects a designated RDP/jump-server host appearing as a Modbus, DNP3, S7comm, or OPC UA protocol client itself rather than simply brokering an interactive remote session, which can indicate an attacker has installed tooling directly on the jump host to issue OT commands. references: - https://attack.mitre.org/techniques/T0867/ tags: - attack.ics.t0867 - attack.ics.t0855 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: generic detection: selection: proto: - modbus - dnp3 - s7comm - opcua src_ip¦in_list: 'designated_jump_hosts' condition: selection falsepositives: - Jump host intentionally repurposed as an engineering access point per an approved architecture change level: critical

## Remote Access Session into L3 Without Approved Change Window

Detects a VPN or jump-server interactive logon to an L3 SCADA/site-operations server occurring outside the standard business hours / approved maintenance window, which should be correlated against the change-management ticketing system before closure.

| Rule ID | d1a1b2c3-0002-4a1b-8c1d-2a2b3c4d5e02 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.t1133, attack.ics.t0859 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: Remote Access Session into L3 Without Approved Change Window id: d1a1b2c3-0002-4a1b-8c1d-2a2b3c4d5e02 status: experimental description: Detects a VPN or jump-server interactive logon to an L3 SCADA/site-operations server occurring outside the standard business hours / approved maintenance window, which should be correlated against the change-management ticketing system before closure. references: - https://attack.mitre.org/techniques/T1133/ tags: - attack.t1133 - attack.ics.t0859 author: Mahesh date: 2026/07/01 logsource: category: authentication product: windows detection: selection: EventID: 4624 LogonType: - 10 - 3 TargetComputerName¦in_list: 'l3_scada_servers' timefilter: hour¦gte: 20 timefilter2: hour¦lte: 5 condition: selection and (timefilter or timefilter2) falsepositives: - Approved emergency maintenance session with an associated change ticket level: medium

## Shared or Service Account Used From Unusual DMZ Source

Detects an engineering or vendor shared/service account authenticating from a DMZ host other than its single documented jump path, indicating credential reuse, theft, or an attacker pivoting using harvested service-account credentials.

| Rule ID | d1a1b2c3-0002-4a1b-8c1d-2a2b3c4d5e03 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.t1078, attack.ics.t0859 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Shared or Service Account Used From Unusual DMZ Source id: d1a1b2c3-0002-4a1b-8c1d-2a2b3c4d5e03 status: experimental description: Detects an engineering or vendor shared/service account authenticating from a DMZ host other than its single documented jump path, indicating credential reuse, theft, or an attacker pivoting using harvested service-account credentials. references: - https://attack.mitre.org/techniques/T1078/ tags: - attack.t1078 - attack.ics.t0859 author: Mahesh date: 2026/07/01 logsource: category: authentication product: windows detection: selection: EventID: 4624 TargetUserName¦in_list: 'shared_service_accounts' filter: WorkstationName¦in_list: 'documented_jump_path_for_account' condition: selection and not filter falsepositives: - Documented jump-path mapping not yet updated after an approved architecture change level: high

## File Transfer From DMZ to L3 With Executable or Engineering Project Extension

Detects an SMB, FTP, or SCP file transfer from an OT DMZ host into an L3 historian or SCADA server where the file extension is an executable, script, or PLC engineering project type (.exe, .dll, .ps1, .acd, .s7p, .ap16, .l5x), occurring outside an approved engineering change window.

| Rule ID | d1a1b2c3-0002-4a1b-8c1d-2a2b3c4d5e04 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0843, attack.t1105 |
| Severity | HIGH |

Sigma Rule (YAML):

title: File Transfer From DMZ to L3 With Executable or Engineering Project Extension id: d1a1b2c3-0002-4a1b-8c1d-2a2b3c4d5e04 status: experimental description: Detects an SMB, FTP, or SCP file transfer from an OT DMZ host into an L3 historian or SCADA server where the file extension is an executable, script, or PLC engineering project type (.exe, .dll, .ps1, .acd, .s7p, .ap16, .l5x), occurring outside an approved engineering change window. references: - https://attack.mitre.org/techniques/T0843/ tags: - attack.ics.t0843 - attack.t1105 author: Mahesh date: 2026/07/01 logsource: category: file_event product: windows detection: selection: TargetFilename¦endswith: - '.exe' - '.dll' - '.ps1' - '.acd' - '.s7p' - '.ap16' - '.l5x' ComputerName¦in_list: 'l3_scada_servers' timefilter: hour¦gte: 20 timefilter2: hour¦lte: 5 condition: selection and (timefilter or timefilter2) falsepositives: - Authorized engineering file deployment during an approved off-hours change window level: high

## Vendor Remote Access Session Duration or Scope Anomaly

Detects a third-party/vendor remote access session exceeding the typical support-call duration baseline, or touching systems outside the vendor's contracted scope, indicating possible session hijacking or scope creep beyond the support agreement.

| Rule ID | d1a1b2c3-0002-4a1b-8c1d-2a2b3c4d5e05 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.t1133, attack.ics.t0883 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: Vendor Remote Access Session Duration or Scope Anomaly id: d1a1b2c3-0002-4a1b-8c1d-2a2b3c4d5e05 status: experimental description: Detects a third-party/vendor remote access session exceeding the typical support-call duration baseline, or touching systems outside the vendor's contracted scope, indicating possible session hijacking or scope creep beyond the support agreement. references: - https://attack.mitre.org/techniques/T1133/ tags: - attack.t1133 - attack.ics.t0883 author: Mahesh date: 2026/07/01 logsource: category: authentication product: vpn detection: selection: account_type: vendor timeframe: 1 condition: selection ¦ duration > 4h falsepositives: - Legitimate extended support engagement for a major upgrade or incident response level: medium

# Site Operations (L3) → Supervisory (L2 — HMI/SCADA)

## Unscheduled HMI Project or Configuration Deployment

Detects an SCADA server pushing a new HMI project/configuration package (WinCC, FactoryTalk View, iFIX) to an HMI client outside the documented change window, which may indicate unauthorized screen, alarm, or logic-display tampering.

| Rule ID | d1a1b2c3-0003-4a1b-8c1d-3a2b3c4d5e01 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0831, attack.ics.t0843 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: Unscheduled HMI Project or Configuration Deployment id: d1a1b2c3-0003-4a1b-8c1d-3a2b3c4d5e01 status: experimental description: Detects an SCADA server pushing a new HMI project/configuration package (WinCC, FactoryTalk View, iFIX) to an HMI client outside the documented change window, which may indicate unauthorized screen, alarm, or logic-display tampering. references: - https://attack.mitre.org/techniques/T0831/ tags: - attack.ics.t0831 - attack.ics.t0843 author: Mahesh date: 2026/07/01 logsource: category: file_event product: windows detection: selection: TargetFilename¦endswith: - '.apa' - '.hmi' - '.fwx' - '.cfg' ComputerName¦in_list: 'l2_hmi_hosts' timefilter: hour¦gte: 20 timefilter2: hour¦lte: 5 condition: selection and (timefilter or timefilter2) falsepositives: - Planned HMI graphics update performed during an approved off-hours window level: medium

## New or Modified Scheduled Task on HMI or SCADA Server

Detects creation or modification of a scheduled task or Windows service on an L2 HMI or L3 SCADA server, a common post-compromise persistence technique observed in CRASHOVERRIDE/Industroyer-class OT intrusions.

| Rule ID | d1a1b2c3-0003-4a1b-8c1d-3a2b3c4d5e02 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.t1053.005, attack.ics.t0839 |
| Severity | HIGH |

Sigma Rule (YAML):

title: New or Modified Scheduled Task on HMI or SCADA Server id: d1a1b2c3-0003-4a1b-8c1d-3a2b3c4d5e02 status: experimental description: Detects creation or modification of a scheduled task or Windows service on an L2 HMI or L3 SCADA server, a common post-compromise persistence technique observed in CRASHOVERRIDE/Industroyer-class OT intrusions. references: - https://attack.mitre.org/techniques/T1053/005/ - https://attack.mitre.org/techniques/T0839/ tags: - attack.t1053.005 - attack.ics.t0839 author: Mahesh date: 2026/07/01 logsource: category: process_creation product: windows detection: selection: Image¦endswith: '\schtasks.exe' CommandLine¦contains: '/create' ComputerName¦in_list: 'l2_l3_ot_servers' condition: selection falsepositives: - Authorized engineering team scheduling a routine maintenance task level: high

## Engineering Software Launched by Non-Engineering Account

Detects PLC/HMI engineering software (Studio 5000, TIA Portal, RSLogix, WinCC Engineering) being launched by a user account that is not a member of the designated engineering role group, indicating a privilege/role mismatch that may signal credential compromise.

| Rule ID | d1a1b2c3-0003-4a1b-8c1d-3a2b3c4d5e03 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0858, attack.t1078 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Engineering Software Launched by Non-Engineering Account id: d1a1b2c3-0003-4a1b-8c1d-3a2b3c4d5e03 status: experimental description: Detects PLC/HMI engineering software (Studio 5000, TIA Portal, RSLogix, WinCC Engineering) being launched by a user account that is not a member of the designated engineering role group, indicating a privilege/role mismatch that may signal credential compromise. references: - https://attack.mitre.org/techniques/T0858/ tags: - attack.ics.t0858 - attack.t1078 author: Mahesh date: 2026/07/01 logsource: category: process_creation product: windows detection: selection: Image¦endswith: - '\Studio5000.exe' - '\TIA Portal.exe' - '\RSLogix5000.exe' - '\WinCCExplorer.exe' filter: User¦in_list: 'engineering_role_group' condition: selection and not filter falsepositives: - Read-only access by a supervisor or auditor account that is intentionally excluded from the engineering group level: high

## Abnormal Authentication Pattern on SCADA or HMI Server

Detects a burst of failed logons (password spray), an off-hours successful logon, or a logon from an account that has never previously authenticated to a given L2/L3 SCADA or HMI server, indicating credential-guessing or use of a freshly compromised account.

| Rule ID | d1a1b2c3-0003-4a1b-8c1d-3a2b3c4d5e04 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.t1110, attack.ics.t0859 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: Abnormal Authentication Pattern on SCADA or HMI Server id: d1a1b2c3-0003-4a1b-8c1d-3a2b3c4d5e04 status: experimental description: Detects a burst of failed logons (password spray), an off-hours successful logon, or a logon from an account that has never previously authenticated to a given L2/L3 SCADA or HMI server, indicating credential-guessing or use of a freshly compromised account. references: - https://attack.mitre.org/techniques/T1110/ tags: - attack.t1110 - attack.ics.t0859 author: Mahesh date: 2026/07/01 logsource: category: authentication product: windows detection: selection_spray: EventID: 4625 TargetComputerName¦in_list: 'l2_l3_ot_servers' timeframe: 10m condition: selection_spray ¦ count() by TargetComputerName > 10 falsepositives: - Misconfigured service account with an outdated stored credential causing repeated failures level: medium

## Unexpected Outbound Connection from L2 HMI Toward DMZ or IT

Detects an L2 HMI host initiating an outbound connection toward the OT DMZ or IT zone. HMIs are typically pure consumers of supervisory data and should not act as connection initiators toward higher zones; this pattern is consistent with C2 beaconing or data staging.

| Rule ID | d1a1b2c3-0003-4a1b-8c1d-3a2b3c4d5e05 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0886, attack.t1041 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Unexpected Outbound Connection from L2 HMI Toward DMZ or IT id: d1a1b2c3-0003-4a1b-8c1d-3a2b3c4d5e05 status: experimental description: Detects an L2 HMI host initiating an outbound connection toward the OT DMZ or IT zone. HMIs are typically pure consumers of supervisory data and should not act as connection initiators toward higher zones; this pattern is consistent with C2 beaconing or data staging. references: - https://attack.mitre.org/techniques/T0886/ tags: - attack.ics.t0886 - attack.t1041 author: Mahesh date: 2026/07/01 logsource: category: firewall product: generic detection: selection: src_zone: L2_HMI dst_zone: - OT_DMZ - IT filter: dst_ip¦in_list: 'approved_l2_outbound_destinations' condition: selection and not filter falsepositives: - Approved historian/alarm-forwarding flow not yet documented in the allow-list level: high

# Supervisory (L2) → Control (L1 — PLC/RTU)

## Control Command From HMI Outside Documented HMI-to-PLC Pairing

Detects a control-protocol command (Modbus/DNP3/S7comm/OPC UA write) issued from an HMI to a PLC/RTU pair that is not present in the documented HMI-to-controller pairing map, e.g. the Line 2 HMI commanding a Line 5 PLC, which can indicate cross-zone pivoting within L2/L1.

| Rule ID | d1a1b2c3-0004-4a1b-8c1d-4a2b3c4d5e01 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0855, attack.ics.t0867 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Control Command From HMI Outside Documented HMI-to-PLC Pairing id: d1a1b2c3-0004-4a1b-8c1d-4a2b3c4d5e01 status: experimental description: Detects a control-protocol command (Modbus/DNP3/S7comm/OPC UA write) issued from an HMI to a PLC/RTU pair that is not present in the documented HMI-to-controller pairing map, e.g. the Line 2 HMI commanding a Line 5 PLC, which can indicate cross-zone pivoting within L2/L1. references: - https://attack.mitre.org/techniques/T0855/ tags: - attack.ics.t0855 - attack.ics.t0867 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: generic detection: selection: proto: - modbus - dnp3 - s7comm - opcua function_code¦exists: true filter: hmi_plc_pair¦in_list: 'documented_hmi_plc_pairing_map' condition: selection and not filter falsepositives: - Newly commissioned process line not yet added to the pairing map level: high

## PLC Logic or Firmware Upload From Non-Engineering Host

Detects a PLC program/firmware download (Modbus, S7comm, EtherNet/IP CIP, or vendor-specific upload service) originating from a host other than the single designated engineering workstation for that process cell.

| Rule ID | d1a1b2c3-0004-4a1b-8c1d-4a2b3c4d5e02 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0843, attack.ics.t0857 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: PLC Logic or Firmware Upload From Non-Engineering Host id: d1a1b2c3-0004-4a1b-8c1d-4a2b3c4d5e02 status: experimental description: Detects a PLC program/firmware download (Modbus, S7comm, EtherNet/IP CIP, or vendor-specific upload service) originating from a host other than the single designated engineering workstation for that process cell. references: - https://attack.mitre.org/techniques/T0843/ tags: - attack.ics.t0843 - attack.ics.t0857 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: generic detection: selection: service: - 'Download block' - 'Request download' - 'firmware_upload' filter: src_ip¦in_list: 'designated_engineering_workstations' condition: selection and not filter falsepositives: - Backup engineering workstation used during primary workstation outage level: critical

## Engineering Protocol Session Opened by HMI Account

Detects an HMI host or HMI service account opening an engineering-class protocol session (S7comm Job/Userdata, EtherNet/IP CIP unconnected message, OMRON FINS) rather than its normal supervisory polling traffic, which can indicate the HMI has been used as a pivot point to reach engineering-level PLC functions.

| Rule ID | d1a1b2c3-0004-4a1b-8c1d-4a2b3c4d5e03 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0867, attack.ics.t0858 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Engineering Protocol Session Opened by HMI Account id: d1a1b2c3-0004-4a1b-8c1d-4a2b3c4d5e03 status: experimental description: Detects an HMI host or HMI service account opening an engineering-class protocol session (S7comm Job/Userdata, EtherNet/IP CIP unconnected message, OMRON FINS) rather than its normal supervisory polling traffic, which can indicate the HMI has been used as a pivot point to reach engineering-level PLC functions. references: - https://attack.mitre.org/techniques/T0867/ tags: - attack.ics.t0867 - attack.ics.t0858 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: generic detection: selection: src_ip¦in_list: 'l2_hmi_hosts' proto: - s7comm - cip - fins rosctr: - 'Job' - 'Userdata' condition: selection falsepositives: - HMI legitimately configured to perform diagnostic reads via engineering protocol services level: high

# Control (L1) → Field Devices (L0)

## Safety Instrumented System Point Forced or Overridden

Detects a force/override flag being set on a tag belonging to the Safety Instrumented System (SIS) address range, a technique directly associated with the TRITON/TRISIS attack against a Schneider Electric Triconex SIS controller.

| Rule ID | d1a1b2c3-0005-4a1b-8c1d-5a2b3c4d5e01 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0837, attack.ics.t0800 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: Safety Instrumented System Point Forced or Overridden id: d1a1b2c3-0005-4a1b-8c1d-5a2b3c4d5e01 status: experimental description: Detects a force/override flag being set on a tag belonging to the Safety Instrumented System (SIS) address range, a technique directly associated with the TRITON/TRISIS attack against a Schneider Electric Triconex SIS controller. references: - https://attack.mitre.org/techniques/T0837/ - https://www.cisa.gov/news-events/ics-advisories tags: - attack.ics.t0837 - attack.ics.t0800 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: generic detection: selection: node_id¦startswith: 'SIS.' write_value_flag: 'force_override' condition: selection falsepositives: - Authorized SIS bypass during a documented maintenance override procedure with management of change sign-off level: critical

## Setpoint Change Exceeding Safe Delta Threshold

Detects a process setpoint (temperature, pressure, flow, level) write where the new value differs from the prior value by more than the site-defined safe-change percentage in a single command, which can indicate an attempt to drive the process outside safe operating limits.

| Rule ID | d1a1b2c3-0005-4a1b-8c1d-5a2b3c4d5e02 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0836, attack.ics.t0855 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Setpoint Change Exceeding Safe Delta Threshold id: d1a1b2c3-0005-4a1b-8c1d-5a2b3c4d5e02 status: experimental description: Detects a process setpoint (temperature, pressure, flow, level) write where the new value differs from the prior value by more than the site-defined safe-change percentage in a single command, which can indicate an attempt to drive the process outside safe operating limits. references: - https://attack.mitre.org/techniques/T0836/ tags: - attack.ics.t0836 - attack.ics.t0855 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: generic detection: selection: node_id¦startswith: 'Setpoint.' condition: selection ¦ delta(write_value, previous_value) > 25percent falsepositives: - Legitimate large process change during startup/shutdown sequencing level: high

## Redundant Sensor Correlation Divergence

Detects the primary and redundant readings for the same physical measurement point (e.g., dual-redundant pressure transmitters) diverging beyond the configured tolerance, which can indicate either an instrumentation fault or a spoofed/manipulated sensor feed used to mask a process manipulation from operators.

| Rule ID | d1a1b2c3-0005-4a1b-8c1d-5a2b3c4d5e03 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0856, attack.ics.t0804 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: Redundant Sensor Correlation Divergence id: d1a1b2c3-0005-4a1b-8c1d-5a2b3c4d5e03 status: experimental description: Detects the primary and redundant readings for the same physical measurement point (e.g., dual-redundant pressure transmitters) diverging beyond the configured tolerance, which can indicate either an instrumentation fault or a spoofed/manipulated sensor feed used to mask a process manipulation from operators. references: - https://attack.mitre.org/techniques/T0856/ tags: - attack.ics.t0856 - attack.ics.t0804 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: generic detection: selection: point_pair_type: 'redundant_sensor' condition: selection ¦ delta(primary_value, redundant_value) > tolerance_threshold falsepositives: - Genuine sensor drift or calibration fault requiring maintenance level: medium

# Cross-Cutting (All Zone Boundaries)

## First-Time Asset Communication Across a Zone Boundary

Detects a source asset establishing a connection across any Purdue Model zone boundary (IT-DMZ, DMZ-L3, L3-L2, L2-L1) for the first time relative to the rolling asset-communication baseline, generalizing the single-protocol "new asset" rule into a cross-zone, cross-protocol behavioral check.

| Rule ID | d1a1b2c3-0006-4a1b-8c1d-6a2b3c4d5e01 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0883, attack.ics.t0846 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: First-Time Asset Communication Across a Zone Boundary id: d1a1b2c3-0006-4a1b-8c1d-6a2b3c4d5e01 status: experimental description: Detects a source asset establishing a connection across any Purdue Model zone boundary (IT-DMZ, DMZ-L3, L3-L2, L2-L1) for the first time relative to the rolling asset-communication baseline, generalizing the single-protocol "new asset" rule into a cross-zone, cross-protocol behavioral check. references: - https://attack.mitre.org/techniques/T0883/ tags: - attack.ics.t0883 - attack.ics.t0846 author: Mahesh date: 2026/07/01 logsource: category: firewall product: generic detection: selection: src_zone¦in_list: 'all_purdue_zones' dst_zone¦in_list: 'all_purdue_zones' filter: src_dst_pair¦in_list: 'baseline_observed_pairs' condition: selection and not filter falsepositives: - Newly commissioned asset pending baseline update level: medium

## Zone Enforcement Firewall Rule Hit-Count Anomaly

Detects a sudden spike in permit-hit count on an inter-zone firewall rule that is historically rarely used, which can indicate exploitation of an overly broad legacy rule to move traffic across a Purdue Model boundary.

| Rule ID | d1a1b2c3-0006-4a1b-8c1d-6a2b3c4d5e02 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0886 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: Zone Enforcement Firewall Rule Hit-Count Anomaly id: d1a1b2c3-0006-4a1b-8c1d-6a2b3c4d5e02 status: experimental description: Detects a sudden spike in permit-hit count on an inter-zone firewall rule that is historically rarely used, which can indicate exploitation of an overly broad legacy rule to move traffic across a Purdue Model boundary. references: - https://attack.mitre.org/techniques/T0886/ tags: - attack.ics.t0886 author: Mahesh date: 2026/07/01 logsource: category: firewall product: generic detection: selection: rule_historical_avg_hits_per_day¦lt: 5 timeframe: 1h condition: selection ¦ count() by rule_id > 50 falsepositives: - Legitimate spike from a scheduled batch job or bulk data transfer level: medium

## Removable Media Event on DMZ or L3 Asset

Detects a USB mass-storage device insertion event on an OT DMZ or L3 SCADA/historian asset, the initial-access vector used by Stuxnet and a recurring technique in OT-targeted intrusions where network segmentation otherwise limits remote access.

| Rule ID | d1a1b2c3-0006-4a1b-8c1d-6a2b3c4d5e03 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0847, attack.t1091 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Removable Media Event on DMZ or L3 Asset id: d1a1b2c3-0006-4a1b-8c1d-6a2b3c4d5e03 status: experimental description: Detects a USB mass-storage device insertion event on an OT DMZ or L3 SCADA/historian asset, the initial-access vector used by Stuxnet and a recurring technique in OT-targeted intrusions where network segmentation otherwise limits remote access. references: - https://attack.mitre.org/techniques/T0847/ tags: - attack.ics.t0847 - attack.t1091 author: Mahesh date: 2026/07/01 logsource: category: device_event product: windows detection: selection: EventID: - 2003 - 2010 ComputerName¦in_list: - 'dmz_jump_hosts' - 'l3_scada_servers' condition: selection falsepositives: - Authorized engineering USB drive used for an approved offline patch/firmware transfer per the site's removable-media policy level: high

## East-West Traffic Between L1 Controllers Across Process Cells

Detects direct peer-to-peer communication between Level 1 PLCs/RTUs belonging to different documented process cells/zones. Controllers should generally only communicate within their own cell via the supervisory layer, not directly with controllers in other cells.

| Rule ID | d1a1b2c3-0006-4a1b-8c1d-6a2b3c4d5e04 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0867, attack.ics.t0883 |
| Severity | HIGH |

Sigma Rule (YAML):

title: East-West Traffic Between L1 Controllers Across Process Cells id: d1a1b2c3-0006-4a1b-8c1d-6a2b3c4d5e04 status: experimental description: Detects direct peer-to-peer communication between Level 1 PLCs/RTUs belonging to different documented process cells/zones. Controllers should generally only communicate within their own cell via the supervisory layer, not directly with controllers in other cells. references: - https://attack.mitre.org/techniques/T0867/ tags: - attack.ics.t0867 - attack.ics.t0883 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: generic detection: selection: src_ip¦in_list: 'l1_controllers' dst_ip¦in_list: 'l1_controllers' filter: src_dst_same_cell: true condition: selection and not filter falsepositives: - Documented inter-cell interlock requiring direct PLC-to-PLC communication level: high

## NTP or Time Desynchronization Anomaly Across IT-OT Path

Detects a host's clock drifting beyond the acceptable tolerance from the authoritative time source, or repeated rejected/failed time-sync attempts, which can be used to desynchronize event timestamps and hinder forensic timeline correlation across the IT-DMZ-OT path during an active intrusion.

| Rule ID | d1a1b2c3-0006-4a1b-8c1d-6a2b3c4d5e05 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0851, attack.ics.t0804 |
| Severity | LOW |

Sigma Rule (YAML):

title: NTP or Time Desynchronization Anomaly Across IT-OT Path id: d1a1b2c3-0006-4a1b-8c1d-6a2b3c4d5e05 status: experimental description: Detects a host's clock drifting beyond the acceptable tolerance from the authoritative time source, or repeated rejected/failed time-sync attempts, which can be used to desynchronize event timestamps and hinder forensic timeline correlation across the IT-DMZ-OT path during an active intrusion. references: - https://attack.mitre.org/techniques/T0851/ tags: - attack.ics.t0851 - attack.ics.t0804 author: Mahesh date: 2026/07/01 logsource: category: system product: generic detection: selection: event_type: 'time_sync_failure' timeframe: 1h condition: selection ¦ count() by host > 5 falsepositives: - Network connectivity issue between a site and its NTP source unrelated to malicious activity level: low

# Deployment & Tuning Notes

- Zone labeling: standardize the src_zone/dst_zone (or equivalent firewall security-zone) values across all enforcement points before import — these rules assume consistent zone naming (IT, OT_DMZ, L3, L2_HMI, L1, L0).

- Reference lists: populate the in_list lookups (approved_jump_host_ips, dmz_historian_ips, designated_engineering_workstations, documented_hmi_plc_pairing_map, l1_controllers, etc.) from the current network segmentation diagram and asset inventory; stale lists are the single biggest source of false positives in this library.

- Pair with the protocol-level library: this cross-zone library is designed to sit alongside the Modbus/DNP3/OPC UA/S7comm/IEC 104 protocol rules — zone-boundary rules catch the access path, protocol rules catch the resulting command.

- Phased rollout: enable IT→DMZ and DMZ→L3 rules first (highest signal, lowest noise), then L3→L2 and L2→L1, then the L1→L0 process-value rules last, since the latter require accurate per-tag safe-delta and tolerance thresholds from process engineering.

- Correlate with change management: several rules (off-hours remote access, off-window file transfer) are designed to be auto-closed or down-graded when correlated against an approved change ticket ID, rather than treated as standalone alerts.

- Safety-tag rules (SIS force/override, setpoint delta) should be reviewed jointly with process safety engineering before tuning thresholds, given the operational and safety sensitivity of these points.