OT/ICS SOC Use Case Library

Sigma Detection Rules — Modbus, DNP3, OPC UA, S7comm, IEC 60870-5-104

Prepared for: Mahesh — OT/ICS SOC, Help AG

Environment: Nozomi Guardian/Vantage, Zeek + ICSNPP protocol parsers

Date: 1 July 2026

Scope & Source Note

SigmaHQ’s public community repository does not maintain a dedicated OT/ICS protocol ruleset — its coverage is overwhelmingly Windows, EDR, and cloud log sources. The rules in this library were authored directly against generic ICS network log fields, written to be compatible with Zeek ICSNPP protocol parser output and Nozomi Guardian/Vantage log exports. Field names (proto, function_code, src_ip, node_id, type_id, etc.) should be mapped to your SIEM’s actual parsed field names before deployment.

Every rule includes a MITRE ATT&CK for ICS tag, a description of the operational risk, false-positive guidance, and a severity level. Replace the placeholder subnets/allow-lists (commented inline in the YAML) with your actual asset inventory before enabling in production.

# Table of Contents

# Use Case Summary

| # | Title | Protocol | ATT&CK ICS | Level |
| --- | --- | --- | --- | --- |
| 1 | Modbus Write Operation from Unauthorized Source | modbus | attack.ics.t0855, attack.ics.t0836 | HIGH |
| 2 | Modbus Diagnostic Restart Communications Function Code | modbus | attack.ics.t0816, attack.ics.t0814 | HIGH |
| 3 | Modbus Exception Response Flood (Possible Scanning or Fuzzing) | modbus | attack.ics.t0846, attack.ics.t0888 | MEDIUM |
| 4 | Modbus Write to Coil/Register Outside Approved Change Window | modbus | attack.ics.t0855, attack.ics.t0831 | MEDIUM |
| 5 | DNP3 Cold or Warm Restart Command Issued | dnp3 | attack.ics.t0816, attack.ics.t0814 | CRITICAL |
| 6 | DNP3 Disable Unsolicited Responses Command | dnp3 | attack.ics.t0804, attack.ics.t0814 | HIGH |
| 7 | DNP3 Direct Operate Without Preceding Select (Bypass of Select-Before-Operate) | dnp3 | attack.ics.t0855, attack.ics.t0858 | HIGH |
| 8 | DNP3 CROB Control Command Outside Maintenance Window | dnp3 | attack.ics.t0855 | MEDIUM |
| 9 | OPC UA Session Established with Anonymous Authentication | opcua | attack.ics.t0859, attack.ics.t0883 | HIGH |
| 10 | OPC UA Untrusted or Self-Signed Certificate Accepted | opcua | attack.ics.t0859, attack.ics.t0822 | HIGH |
| 11 | OPC UA Write Request to Critical Node from Unauthorized Client | opcua | attack.ics.t0855, attack.ics.t0836 | HIGH |
| 12 | OPC UA Address Space Browse Flood (Reconnaissance) | opcua | attack.ics.t0846, attack.ics.t0888 | MEDIUM |
| 13 | S7comm PLC STOP Command Issued | s7comm | attack.ics.t0816, attack.ics.t0858 | CRITICAL |
| 14 | S7comm Program Block Download/Upload Outside Change Window | s7comm | attack.ics.t0843, attack.ics.t0871 | HIGH |
| 15 | S7comm System Status List (SZL) Enumeration | s7comm | attack.ics.t0888, attack.ics.t0846 | MEDIUM |
| 16 | IEC 60870-5-104 General Interrogation from Unauthorized Master | iec104 | attack.ics.t0846, attack.ics.t0888 | HIGH |
| 17 | IEC 60870-5-104 Single or Double Command Outside Maintenance Window | iec104 | attack.ics.t0855 | MEDIUM |
| 18 | IEC 60870-5-104 Clock Synchronization Command Anomaly | iec104 | attack.ics.t0804, attack.ics.t0851 | LOW |
| 19 | Engineering Workstation Communicating with Multiple PLCs Across Zones | generic | attack.ics.t0867, attack.ics.t0843 | HIGH |
| 20 | New or Unauthorized Asset Communicating on OT Engineering Protocol Port | generic | attack.ics.t0883, attack.ics.t0846 | MEDIUM |

# Modbus (RTU / ASCII / TCP)

## Modbus Write Operation from Unauthorized Source

Detects Modbus write function codes (05,06,0F,10) issued by a source IP that is not in the known engineering workstation/SCADA master allow-list.

| Rule ID | 7a1e9b3a-1d2c-4e1a-9b4e-1a2b3c4d5e01 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0855, attack.ics.t0836 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Modbus Write Operation from Unauthorized Source id: 7a1e9b3a-1d2c-4e1a-9b4e-1a2b3c4d5e01 status: experimental description: Detects Modbus write function codes (05,06,0F,10) issued by a source IP that is not in the known engineering workstation/SCADA master allow-list. references: - https://www.modbus.org/docs/Modbus_Application_Protocol_V1_1b3.pdf tags: - attack.ics.t0855 - attack.ics.t0836 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: modbus detection: selection: proto: modbus function_code: - 5 - 6 - 15 - 16 filter: src_ip¦cidr: '10.10.10.0/24' # replace with engineering workstation/HMI subnet condition: selection and not filter falsepositives: - Newly commissioned engineering workstation not yet added to allow-list level: high

## Modbus Diagnostic Restart Communications Function Code

Detects Modbus Diagnostics (function code 08) sub-function 01 (Restart Communications Option), which forces a slave device to reinitialize its communication port — can be used as a denial-of-service technique against a PLC/RTU.

| Rule ID | 7a1e9b3a-1d2c-4e1a-9b4e-1a2b3c4d5e02 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0816, attack.ics.t0814 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Modbus Diagnostic Restart Communications Function Code id: 7a1e9b3a-1d2c-4e1a-9b4e-1a2b3c4d5e02 status: experimental description: Detects Modbus Diagnostics (function code 08) sub-function 01 (Restart Communications Option), which forces a slave device to reinitialize its communication port — can be used as a denial-of-service technique against a PLC/RTU. references: - https://www.modbus.org/docs/Modbus_Application_Protocol_V1_1b3.pdf tags: - attack.ics.t0816 - attack.ics.t0814 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: modbus detection: selection: proto: modbus function_code: 8 sub_function_code: 1 condition: selection falsepositives: - Scheduled maintenance window restart performed by engineering team level: high

## Modbus Exception Response Flood (Possible Scanning or Fuzzing)

Detects a high volume of Modbus exception responses (function code with high bit set / exception codes 01-04) from a single slave within a short window, indicating function code probing, fuzzing, or protocol scanning.

| Rule ID | 7a1e9b3a-1d2c-4e1a-9b4e-1a2b3c4d5e03 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0846, attack.ics.t0888 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: Modbus Exception Response Flood (Possible Scanning or Fuzzing) id: 7a1e9b3a-1d2c-4e1a-9b4e-1a2b3c4d5e03 status: experimental description: Detects a high volume of Modbus exception responses (function code with high bit set / exception codes 01-04) from a single slave within a short window, indicating function code probing, fuzzing, or protocol scanning. references: - https://www.modbus.org/docs/Modbus_Application_Protocol_V1_1b3.pdf tags: - attack.ics.t0846 - attack.ics.t0888 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: modbus detection: selection: proto: modbus exception_code: - 1 - 2 - 3 - 4 timeframe: 5m condition: selection ¦ count(exception_code) by src_ip, dst_ip > 20 falsepositives: - Faulty HMI polling configuration causing repeated invalid requests level: medium

## Modbus Write to Coil/Register Outside Approved Change Window

Detects Modbus write function codes occurring outside the defined maintenance/change window, which may indicate unauthorized process manipulation.

| Rule ID | 7a1e9b3a-1d2c-4e1a-9b4e-1a2b3c4d5e04 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0855, attack.ics.t0831 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: Modbus Write to Coil/Register Outside Approved Change Window id: 7a1e9b3a-1d2c-4e1a-9b4e-1a2b3c4d5e04 status: experimental description: Detects Modbus write function codes occurring outside the defined maintenance/change window, which may indicate unauthorized process manipulation. references: - https://attack.mitre.org/techniques/T0855/ tags: - attack.ics.t0855 - attack.ics.t0831 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: modbus detection: selection: proto: modbus function_code: - 5 - 6 - 15 - 16 timefilter: hour¦gte: 20 timefilter2: hour¦lte: 5 condition: selection and (timefilter or timefilter2) falsepositives: - Legitimate off-hours maintenance with an approved change ticket level: medium

# DNP3

## DNP3 Cold or Warm Restart Command Issued

Detects DNP3 function code 13 (Cold Restart) or 14 (Warm Restart), which forces an outstation/RTU to reboot. High operational-impact command rarely used outside planned maintenance.

| Rule ID | 8b2f0c4b-2e3d-4f2b-ac5f-2b3c4d5e6f01 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0816, attack.ics.t0814 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: DNP3 Cold or Warm Restart Command Issued id: 8b2f0c4b-2e3d-4f2b-ac5f-2b3c4d5e6f01 status: experimental description: Detects DNP3 function code 13 (Cold Restart) or 14 (Warm Restart), which forces an outstation/RTU to reboot. High operational-impact command rarely used outside planned maintenance. references: - https://www.dnp.org/About/Overview-of-DNP3-Protocol tags: - attack.ics.t0816 - attack.ics.t0814 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: dnp3 detection: selection: proto: dnp3 function_code: - 13 - 14 condition: selection falsepositives: - Scheduled outstation maintenance/firmware update level: critical

## DNP3 Disable Unsolicited Responses Command

Detects DNP3 function code 21 (Disable Unsolicited Responses), which can be abused to suppress event/alarm reporting from an outstation to the master, masking subsequent malicious activity.

| Rule ID | 8b2f0c4b-2e3d-4f2b-ac5f-2b3c4d5e6f02 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0804, attack.ics.t0814 |
| Severity | HIGH |

Sigma Rule (YAML):

title: DNP3 Disable Unsolicited Responses Command id: 8b2f0c4b-2e3d-4f2b-ac5f-2b3c4d5e6f02 status: experimental description: Detects DNP3 function code 21 (Disable Unsolicited Responses), which can be abused to suppress event/alarm reporting from an outstation to the master, masking subsequent malicious activity. references: - https://www.dnp.org/About/Overview-of-DNP3-Protocol tags: - attack.ics.t0804 - attack.ics.t0814 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: dnp3 detection: selection: proto: dnp3 function_code: 21 condition: selection falsepositives: - Planned reconfiguration of polling/reporting model by engineering team level: high

## DNP3 Direct Operate Without Preceding Select (Bypass of Select-Before-Operate)

Detects DNP3 Direct Operate (function code 5) issued to a CROB point without a preceding Select (function code 3) for the same point within a short window. Bypassing select-before-operate removes the safety check designed to prevent erroneous control actions.

| Rule ID | 8b2f0c4b-2e3d-4f2b-ac5f-2b3c4d5e6f03 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0855, attack.ics.t0858 |
| Severity | HIGH |

Sigma Rule (YAML):

title: DNP3 Direct Operate Without Preceding Select (Bypass of Select-Before-Operate) id: 8b2f0c4b-2e3d-4f2b-ac5f-2b3c4d5e6f03 status: experimental description: Detects DNP3 Direct Operate (function code 5) issued to a CROB point without a preceding Select (function code 3) for the same point within a short window. Bypassing select-before-operate removes the safety check designed to prevent erroneous control actions. references: - https://www.dnp.org/About/Overview-of-DNP3-Protocol tags: - attack.ics.t0855 - attack.ics.t0858 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: dnp3 detection: operate: proto: dnp3 function_code: 5 select: proto: dnp3 function_code: 3 condition: operate and not near(select, field=point_index, window=10s) falsepositives: - Outstation/master implementation that intentionally uses Direct Operate only, by design level: high

## DNP3 CROB Control Command Outside Maintenance Window

Detects DNP3 Control Relay Output Block (CROB) commands (function codes 3,4,5,6) issued from the SCADA master outside the approved operating/maintenance window, which may indicate unauthorized switching/breaker operation.

| Rule ID | 8b2f0c4b-2e3d-4f2b-ac5f-2b3c4d5e6f04 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0855 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: DNP3 CROB Control Command Outside Maintenance Window id: 8b2f0c4b-2e3d-4f2b-ac5f-2b3c4d5e6f04 status: experimental description: Detects DNP3 Control Relay Output Block (CROB) commands (function codes 3,4,5,6) issued from the SCADA master outside the approved operating/maintenance window, which may indicate unauthorized switching/breaker operation. references: - https://www.dnp.org/About/Overview-of-DNP3-Protocol tags: - attack.ics.t0855 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: dnp3 detection: selection: proto: dnp3 function_code: - 3 - 4 - 5 - 6 object_group: 12 # CROB object group timefilter: hour¦gte: 20 timefilter2: hour¦lte: 5 condition: selection and (timefilter or timefilter2) falsepositives: - Emergency switching operation performed by authorized operator off-hours level: medium

# OPC UA

## OPC UA Session Established with Anonymous Authentication

Detects an OPC UA session activated using the Anonymous UserIdentityToken policy against a server, which should not be permitted on a production OT OPC UA server per IEC 62443 hardening guidance.

| Rule ID | 9c3a1d5c-3f4e-405c-bd6a-3c4d5e6f7001 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0859, attack.ics.t0883 |
| Severity | HIGH |

Sigma Rule (YAML):

title: OPC UA Session Established with Anonymous Authentication id: 9c3a1d5c-3f4e-405c-bd6a-3c4d5e6f7001 status: experimental description: Detects an OPC UA session activated using the Anonymous UserIdentityToken policy against a server, which should not be permitted on a production OT OPC UA server per IEC 62443 hardening guidance. references: - https://reference.opcfoundation.org/Core/Part4/v104/docs/5.6.3 tags: - attack.ics.t0859 - attack.ics.t0883 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: opcua detection: selection: proto: opcua service: ActivateSession user_identity_token_type: Anonymous condition: selection falsepositives: - Lab/test OPC UA server intentionally configured for anonymous access level: high

## OPC UA Untrusted or Self-Signed Certificate Accepted

Detects an OPC UA secure channel established where the client or server certificate was not present in the trusted certificate store, indicating certificate validation has been bypassed or set to AutoAccept.

| Rule ID | 9c3a1d5c-3f4e-405c-bd6a-3c4d5e6f7002 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0859, attack.ics.t0822 |
| Severity | HIGH |

Sigma Rule (YAML):

title: OPC UA Untrusted or Self-Signed Certificate Accepted id: 9c3a1d5c-3f4e-405c-bd6a-3c4d5e6f7002 status: experimental description: Detects an OPC UA secure channel established where the client or server certificate was not present in the trusted certificate store, indicating certificate validation has been bypassed or set to AutoAccept. references: - https://reference.opcfoundation.org/Core/Part2/v104/docs/4.8 tags: - attack.ics.t0859 - attack.ics.t0822 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: opcua detection: selection: proto: opcua service: OpenSecureChannel certificate_trusted: false condition: selection falsepositives: - Newly deployed OPC UA client/server pending certificate exchange during commissioning level: high

## OPC UA Write Request to Critical Node from Unauthorized Client

Detects an OPC UA Write service call targeting a NodeId within the critical control address-space range from a client IP not in the approved SCADA/HMI allow-list.

| Rule ID | 9c3a1d5c-3f4e-405c-bd6a-3c4d5e6f7003 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0855, attack.ics.t0836 |
| Severity | HIGH |

Sigma Rule (YAML):

title: OPC UA Write Request to Critical Node from Unauthorized Client id: 9c3a1d5c-3f4e-405c-bd6a-3c4d5e6f7003 status: experimental description: Detects an OPC UA Write service call targeting a NodeId within the critical control address-space range from a client IP not in the approved SCADA/HMI allow-list. references: - https://reference.opcfoundation.org/Core/Part4/v104/docs/5.10.4 tags: - attack.ics.t0855 - attack.ics.t0836 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: opcua detection: selection: proto: opcua service: Write node_id¦startswith: 'ns=2;s=Control.' filter: src_ip¦cidr: '10.10.10.0/24' # replace with approved HMI/SCADA subnet condition: selection and not filter falsepositives: - Newly added HMI client not yet added to the allow-list level: high

## OPC UA Address Space Browse Flood (Reconnaissance)

Detects a high volume of OPC UA Browse service calls from a single client within a short window, consistent with automated enumeration of the server's address space/tag list prior to a targeted attack.

| Rule ID | 9c3a1d5c-3f4e-405c-bd6a-3c4d5e6f7004 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0846, attack.ics.t0888 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: OPC UA Address Space Browse Flood (Reconnaissance) id: 9c3a1d5c-3f4e-405c-bd6a-3c4d5e6f7004 status: experimental description: Detects a high volume of OPC UA Browse service calls from a single client within a short window, consistent with automated enumeration of the server's address space/tag list prior to a targeted attack. references: - https://reference.opcfoundation.org/Core/Part4/v104/docs/5.8.2 tags: - attack.ics.t0846 - attack.ics.t0888 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: opcua detection: selection: proto: opcua service: Browse timeframe: 5m condition: selection ¦ count() by src_ip > 100 falsepositives: - Engineering tool performing full project tag synchronization level: medium

# S7comm

## S7comm PLC STOP Command Issued

Detects an S7comm PLC control function requesting STOP, which halts CPU program execution. A high-impact command that should only occur during a planned, authorized maintenance action.

| Rule ID | ad4b2e6d-405f-416d-ce7b-4d5e6f708101 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0816, attack.ics.t0858 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: S7comm PLC STOP Command Issued id: ad4b2e6d-405f-416d-ce7b-4d5e6f708101 status: experimental description: Detects an S7comm PLC control function requesting STOP, which halts CPU program execution. A high-impact command that should only occur during a planned, authorized maintenance action. references: - https://wiki.wireshark.org/S7comm tags: - attack.ics.t0816 - attack.ics.t0858 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: s7comm detection: selection: proto: s7comm rosctr: 'Userdata' function_group: 'CPU functions' subfunction: 'PLC Stop' condition: selection falsepositives: - Authorized PLC programmer halting CPU for an approved logic change level: critical

## S7comm Program Block Download/Upload Outside Change Window

Detects S7comm Download/Upload service requests (block write/read of OB/FB/FC/DB blocks) occurring outside the approved engineering change window, indicating possible unauthorized PLC logic modification.

| Rule ID | ad4b2e6d-405f-416d-ce7b-4d5e6f708102 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0843, attack.ics.t0871 |
| Severity | HIGH |

Sigma Rule (YAML):

title: S7comm Program Block Download/Upload Outside Change Window id: ad4b2e6d-405f-416d-ce7b-4d5e6f708102 status: experimental description: Detects S7comm Download/Upload service requests (block write/read of OB/FB/FC/DB blocks) occurring outside the approved engineering change window, indicating possible unauthorized PLC logic modification. references: - https://wiki.wireshark.org/S7comm tags: - attack.ics.t0843 - attack.ics.t0871 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: s7comm detection: selection: proto: s7comm rosctr: 'Job' function: - 'Request download' - 'Download block' - 'Request upload' timefilter: hour¦gte: 20 timefilter2: hour¦lte: 5 condition: selection and (timefilter or timefilter2) falsepositives: - Emergency logic fix performed off-hours with an approved change ticket level: high

## S7comm System Status List (SZL) Enumeration

Detects repeated S7comm "Read SZL" (System Zustands Liste) requests used to enumerate PLC module identification, firmware version, and diagnostic data — a common reconnaissance step preceding targeted exploitation (e.g., as used by Industroyer2/INCONTROLLER-class tooling).

| Rule ID | ad4b2e6d-405f-416d-ce7b-4d5e6f708103 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0888, attack.ics.t0846 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: S7comm System Status List (SZL) Enumeration id: ad4b2e6d-405f-416d-ce7b-4d5e6f708103 status: experimental description: Detects repeated S7comm "Read SZL" (System Zustands Liste) requests used to enumerate PLC module identification, firmware version, and diagnostic data — a common reconnaissance step preceding targeted exploitation (e.g., as used by Industroyer2/INCONTROLLER-class tooling). references: - https://wiki.wireshark.org/S7comm - https://www.cisa.gov/news-events/alerts tags: - attack.ics.t0888 - attack.ics.t0846 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: s7comm detection: selection: proto: s7comm rosctr: 'Userdata' function_group: 'CPU functions' subfunction: 'Read SZL' timeframe: 10m condition: selection ¦ count() by src_ip, dst_ip > 15 falsepositives: - Engineering workstation performing legitimate diagnostic/asset inventory scan level: medium

# IEC 60870-5-104

## IEC 60870-5-104 General Interrogation from Unauthorized Master

Detects an IEC 104 General Interrogation command (Type ID C_IC_NA_1) originating from a source IP that is not the approved control center/SCADA master, which can be used to mass-poll RTU/IED status as a reconnaissance or pre-positioning step.

| Rule ID | be5c3f7e-516f-427e-df8c-5e6f70819201 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0846, attack.ics.t0888 |
| Severity | HIGH |

Sigma Rule (YAML):

title: IEC 60870-5-104 General Interrogation from Unauthorized Master id: be5c3f7e-516f-427e-df8c-5e6f70819201 status: experimental description: Detects an IEC 104 General Interrogation command (Type ID C_IC_NA_1) originating from a source IP that is not the approved control center/SCADA master, which can be used to mass-poll RTU/IED status as a reconnaissance or pre-positioning step. references: - https://www.iec.ch/standards tags: - attack.ics.t0846 - attack.ics.t0888 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: iec104 detection: selection: proto: iec104 type_id: 'C_IC_NA_1' filter: src_ip¦cidr: '10.10.20.0/24' # replace with approved control center subnet condition: selection and not filter falsepositives: - Newly commissioned backup control center not yet allow-listed level: high

## IEC 60870-5-104 Single or Double Command Outside Maintenance Window

Detects IEC 104 Single Command (C_SC_NA_1) or Double Command (C_DC_NA_1) operate requests issued outside the approved maintenance/switching window, which may indicate an unauthorized breaker/switch operation.

| Rule ID | be5c3f7e-516f-427e-df8c-5e6f70819202 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0855 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: IEC 60870-5-104 Single or Double Command Outside Maintenance Window id: be5c3f7e-516f-427e-df8c-5e6f70819202 status: experimental description: Detects IEC 104 Single Command (C_SC_NA_1) or Double Command (C_DC_NA_1) operate requests issued outside the approved maintenance/switching window, which may indicate an unauthorized breaker/switch operation. references: - https://www.iec.ch/standards tags: - attack.ics.t0855 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: iec104 detection: selection: proto: iec104 type_id: - 'C_SC_NA_1' - 'C_DC_NA_1' timefilter: hour¦gte: 20 timefilter2: hour¦lte: 5 condition: selection and (timefilter or timefilter2) falsepositives: - Emergency switching operation by authorized dispatcher off-hours level: medium

## IEC 60870-5-104 Clock Synchronization Command Anomaly

Detects IEC 104 Clock Synchronization commands (C_CS_NA_1) sent at an abnormal frequency or from a non-master source, which can be used to desynchronize event timestamps and hinder forensic timeline reconstruction.

| Rule ID | be5c3f7e-516f-427e-df8c-5e6f70819203 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0804, attack.ics.t0851 |
| Severity | LOW |

Sigma Rule (YAML):

title: IEC 60870-5-104 Clock Synchronization Command Anomaly id: be5c3f7e-516f-427e-df8c-5e6f70819203 status: experimental description: Detects IEC 104 Clock Synchronization commands (C_CS_NA_1) sent at an abnormal frequency or from a non-master source, which can be used to desynchronize event timestamps and hinder forensic timeline reconstruction. references: - https://www.iec.ch/standards tags: - attack.ics.t0804 - attack.ics.t0851 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: iec104 detection: selection: proto: iec104 type_id: 'C_CS_NA_1' timeframe: 1h condition: selection ¦ count() by dst_ip > 5 falsepositives: - Misconfigured NTP/time sync causing repeated correction commands level: low

# Cross-Protocol / Asset & Network Behavior

## Engineering Workstation Communicating with Multiple PLCs Across Zones

Detects a single engineering workstation host establishing OT protocol sessions (Modbus/S7comm/DNP3/OPC UA) with PLCs/RTUs/outstations across more Purdue Model zones or process cells than its documented baseline, indicating possible lateral movement using a compromised EWS.

| Rule ID | cf6d4f8f-627a-438f-e09d-6f708192a301 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0867, attack.ics.t0843 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Engineering Workstation Communicating with Multiple PLCs Across Zones id: cf6d4f8f-627a-438f-e09d-6f708192a301 status: experimental description: Detects a single engineering workstation host establishing OT protocol sessions (Modbus/S7comm/DNP3/OPC UA) with PLCs/RTUs/outstations across more Purdue Model zones or process cells than its documented baseline, indicating possible lateral movement using a compromised EWS. references: - https://attack.mitre.org/techniques/T0867/ tags: - attack.ics.t0867 - attack.ics.t0843 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: generic detection: selection: proto: - modbus - s7comm - dnp3 - opcua src_ip¦cidr: '10.10.10.0/24' # engineering workstation subnet timeframe: 1h condition: selection ¦ count(dst_ip) by src_ip > 8 falsepositives: - Site-wide engineering activity such as firmware rollout or scheduled audit level: high

## New or Unauthorized Asset Communicating on OT Engineering Protocol Port

Detects a source IP not previously observed in the OT asset inventory initiating a session on a known industrial protocol port (Modbus/502, DNP3/20000, S7comm/102, OPC UA/4840, IEC104/2404), indicating rogue device connection or unauthorized network access.

| Rule ID | cf6d4f8f-627a-438f-e09d-6f708192a302 |
| --- | --- |
| Status | experimental |
| MITRE ATT&CK ICS | attack.ics.t0883, attack.ics.t0846 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: New or Unauthorized Asset Communicating on OT Engineering Protocol Port id: cf6d4f8f-627a-438f-e09d-6f708192a302 status: experimental description: Detects a source IP not previously observed in the OT asset inventory initiating a session on a known industrial protocol port (Modbus/502, DNP3/20000, S7comm/102, OPC UA/4840, IEC104/2404), indicating rogue device connection or unauthorized network access. references: - https://attack.mitre.org/techniques/T0883/ tags: - attack.ics.t0883 - attack.ics.t0846 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: generic detection: selection: dst_port: - 502 - 20000 - 102 - 4840 - 2404 filter: src_ip¦in_list: 'known_ot_asset_inventory' # reference to asset inventory lookup list condition: selection and not filter falsepositives: - Newly commissioned asset pending addition to inventory/CMDB level: medium

# Deployment & Tuning Notes

- Field mapping: confirm exact field names emitted by your log source (Nozomi syslog/CEF export vs. Zeek ICSNPP JSON) and update each rule’s detection fields accordingly before import into the SIEM.

- Allow-lists: rules referencing src_ip|cidr or in_list placeholders must be populated from the live OT asset inventory / Purdue Model zone documentation prior to enabling in “high” or “critical” alerting tiers.

- Time-window rules: the 20:00–05:00 maintenance-window filters are examples — replace with the site’s actual approved change-window schedule.

- Baselining: run all rules in a passive/log-only mode for 2–4 weeks per site before promoting to active alerting, given the high environment-to-environment variability of legitimate engineering traffic.

- MITRE ATT&CK for ICS tags follow the attack.ics.<technique-id> namespace convention; cross-reference against your detection coverage heat map (Nozomi Guardian/Vantage use-case matrix) to avoid duplicate alerting.