OT Threat Actor & Historical Attack Use Case Library

Sigma Detection Rules Mapped to Sandworm, APT34, APT33, CHERNOVITE, and XENOTIME TTPs

Prepared for: Mahesh — OT/ICS SOC, Help AG

Environment: Windows/AD, Firewall, Nozomi Guardian/Vantage, Zeek + ICSNPP

Date: 1 July 2026

Purpose & Use

This library translates the publicly documented tactics, techniques, and procedures (TTPs) of the threat actors and malware families most relevant to a Gulf-region OT SOC into deployable Sigma detection logic. It complements the protocol-level library (Modbus/DNP3/OPC UA/S7comm/IEC 104) and the IT→DMZ→OT cross-zone library by adding the attacker behavioral context: who is known to target this region and this sector, and what specific sequence of actions their tooling performs.

The threat_actor field on each rule is informational metadata for triage and threat-intel correlation; it is not part of the core Sigma specification and can be mapped into your SIEM’s case-management or threat-intel platform as a custom tag.

# Table of Contents

# Threat Actor Profiles

| Actor | Attribution | Primary Targets | Notable Incidents | Why It Matters |
| --- | --- | --- | --- | --- |
| Sandworm / APT44 | Russia (GRU Unit 74455) | Ukrainian and allied critical infrastructure; energy, government | BlackEnergy3 (2015 Ukraine grid outage), Industroyer/CRASHOVERRIDE (2016), NotPetya (2017), Industroyer2 (2022), FrostyGoop (2024 Lviv heating) | Only publicly confirmed Tier-1 actor with a track record of causing physical OT impact via cyberattack; uses both custom ICS malware (IEC-104/Modbus/IEC-61850 modules) and destructive wipers to cover tracks. |
| APT34 / OilRig | Iran (MOIS) | Middle East government, energy, telecom, financial services; long-dwell espionage | Helminth/QUADAGENT/Outer Space backdoors; 2024–26 campaigns against Iraqi government and Israeli critical infrastructure | Primarily an espionage and long-term access actor rather than a destructive one; builds persistent footholds (web shells, DNS C2) that can be handed off or escalated toward OT-adjacent systems. |
| APT33 / Elfin | Iran (assessed) | Saudi Arabia, UAE, and US aerospace, energy, and petrochemical sectors | Shamoon V2/V3 and Filerase destructive wiper attacks (2012, 2016–18) against Saudi Aramco-linked and Gulf energy organizations | Known for password spraying and supply-chain compromise as initial access, followed by mass deployment of disk-wiping malware via PsExec-based spreaders. |
| CHERNOVITE (PIPEDREAM / INCONTROLLER) | Not formally attributed | Vendor-agnostic: Schneider Electric Modicon, OMRON Sysmac NEX, OPC UA, CODESYS-based controllers | Discovered pre-deployment in 2022 (CISA AA22-103A); not yet used in a confirmed real-world attack | Assessed by Dragos as an "effects/impact" toolkit built for an access team to hand off to — the first known reusable, modular, multi-vendor ICS attack framework. |
| XENOTIME (TRITON / TRISIS) | Russia (Central Scientific Research Institute, assessed) | Oil & gas and LNG, with continued reconnaissance against ONG component manufacturers | TRITON/TRISIS attack on a Saudi petrochemical plant’s Schneider Triconex Safety Instrumented System (2017) | The only known attack to specifically target a Safety Instrumented System with the intent to disable protective functions, raising the risk of physical harm rather than just process disruption. |

# Known OT Cyberattack Timeline

| Year | Incident | Target | Summary |
| --- | --- | --- | --- |
| 2010 | Stuxnet | Iran (Natanz) | PLC rootkit manipulating centrifuge speed while showing normal values to operators; first confirmed cyber-physical sabotage weapon. |
| 2014 | BlackEnergy2/Havex | European/US energy sector | ICS reconnaissance and HMI data-harvesting malware preceding the 2015 Ukraine attack. |
| 2015 | BlackEnergy3 + KillDisk | Ukraine power grid (3 oblenergos) | Spearphishing → credential theft → remote breaker opening via compromised HMI → KillDisk wiper to delay recovery; first confirmed cyberattack to cause a power outage. |
| 2016 | Industroyer / CRASHOVERRIDE | Ukraine power grid (Kyiv) | First malware with native IEC-101/104/61850/OPC modules built to directly speak substation protocols. |
| 2017 | TRITON / TRISIS | Saudi petrochemical plant | First attack to target a Safety Instrumented System (Schneider Triconex) with intent to disable protective shutdown logic. |
| 2017 | NotPetya | Global (Maersk, Merck, etc.) | Destructive wiper disguised as ransomware, spread via compromised accounting software update; caused billions in damage including OT-adjacent shutdowns. |
| 2019 | LockerGoga | Norsk Hydro | Ransomware forced a precautionary switch to manual OT operations across multiple aluminum plants. |
| 2021 | Oldsmar Water Treatment Intrusion | Florida, USA | Remote-access (TeamViewer) intrusion attempted to raise sodium hydroxide levels in the water supply; caught by an operator watching the cursor move. |
| 2021 | Colonial Pipeline (DarkSide) | US fuel pipeline | IT-side ransomware caused a precautionary full pipeline shutdown despite no direct OT compromise, demonstrating IT/OT interdependency risk. |
| 2022 | Industroyer2 | Ukraine power grid | Sandworm/Electrum reused and refined the Industroyer IEC-104 module for a second grid-disruption attempt, paired with wiper malware to cover tracks. |
| 2022 | PIPEDREAM / INCONTROLLER | Discovered pre-deployment | Modular, multi-vendor ICS attack toolkit (CHERNOVITE) capable of targeting Schneider, OMRON, OPC UA, and CODESYS devices; never confirmed deployed. |
| 2024 | FrostyGoop | Lviv, Ukraine (district heating) | First malware confirmed to use native Modbus TCP for direct device sabotage; cut heating to 600+ buildings in winter. |

# Use Case Summary

| # | Title | Threat Actor | ATT&CK | Level |
| --- | --- | --- | --- | --- |
| 1 | IEC 104 Mass Coordinated Breaker-Open Command Burst | Sandworm / APT44 (Electrum) | attack.ics.t0855, attack.ics.t0831, attack.ics.t0879 | CRITICAL |
| 2 | Modbus Holding Register Manipulation at Abnormal Cadence | Sandworm / APT44 (assessed) | attack.ics.t0855, attack.ics.t0836 | HIGH |
| 3 | Mass Disk-Wiping Utility Execution Across Multiple Hosts | Sandworm / APT44 | attack.t1490, attack.t1561, attack.ics.t0826 | CRITICAL |
| 4 | IEC 61850 GOOSE Unauthorized Publisher or Spoofed Message | Sandworm / APT44 (Electrum) | attack.ics.t0855, attack.ics.t0856 | CRITICAL |
| 5 | Mass Lateral Deployment Tooling Preceding Suspected Wiper Activity | Sandworm / APT44 | attack.t1570, attack.t1047, attack.ics.t0867 | HIGH |
| 6 | DNS Tunneling Beacon From OT DMZ or Engineering Host | APT34 / OilRig (Helix Kitten) | attack.t1071.004, attack.t1568 | HIGH |
| 7 | Web Shell Activity on Internet-Facing OT-Adjacent Server | APT34 / OilRig (Crambus) | attack.t1505.003, attack.t1190 | CRITICAL |
| 8 | Credential Harvesting Tool Execution on Engineering or SCADA Host | APT34 / OilRig | attack.t1003.001, attack.t1003.002, attack.ics.t0859 | CRITICAL |
| 9 | Low-and-Slow HTTPS Beaconing Indicative of OilRig-Class Backdoor C2 | APT34 / OilRig | attack.t1071.001, attack.t1102 | MEDIUM |
| 10 | Shamoon-Style MBR Wiper Execution | APT33 / Elfin (Refined Kitten) | attack.t1561.002, attack.t1529, attack.ics.t0826 | CRITICAL |
| 11 | Password Spray Against OT Remote Access Portal | APT33 / Elfin | attack.t1110.003, attack.t1133 | HIGH |
| 12 | Mass Malware Spreader Tool Execution Across Multiple Hosts | APT33 / Elfin | attack.t1570, attack.t1021.002 | CRITICAL |
| 13 | OPC UA Server Enumeration Followed by Node Value Tampering | CHERNOVITE (PIPEDREAM / INCONTROLLER) | attack.ics.t0846, attack.ics.t0856 | HIGH |
| 14 | CODESYS Protocol Command to Schneider Controller From Non-Engineering Host | CHERNOVITE (PIPEDREAM / INCONTROLLER) | attack.ics.t0859, attack.ics.t0843, attack.ics.t0814 | CRITICAL |
| 15 | OMRON FINS/HTTP Backdoor Command Execution | CHERNOVITE (PIPEDREAM / INCONTROLLER) | attack.ics.t0859, attack.ics.t0883 | HIGH |
| 16 | ASRock RGB Driver Exploitation Attempt (CVE-2020-15368) | CHERNOVITE (PIPEDREAM / INCONTROLLER) | attack.t1068, attack.t1562.001 | CRITICAL |
| 17 | Triconex Safety Controller Engineering Session From Unauthorized Host | XENOTIME (TRITON / TRISIS) | attack.ics.t0859, attack.ics.t0800, attack.ics.t0837 | CRITICAL |
| 18 | Safety Controller Key-Switch or Mode Change Outside Proof-Test Window | XENOTIME (TRITON / TRISIS) | attack.ics.t0858, attack.ics.t0800 | CRITICAL |
| 19 | Engineering Workstation Reflective/Memory-Only Process Injection | Generalized (Stuxnet-class TTP) | attack.t1055, attack.ics.t0851 | HIGH |
| 20 | Mass File Encryption or Rename Activity on OT Historian or Engineering Host | Generalized (Ransomware precursor to OT shutdown) | attack.t1486, attack.ics.t0826 | CRITICAL |

# Sandworm / APT44 (Electrum, BlackEnergy, Voodoo Bear)

## IEC 104 Mass Coordinated Breaker-Open Command Burst

Detects multiple IEC 60870-5-104 Single/Double Command (open) operations issued to several distinct substation RTUs within a short time window from the same or correlated source(s), matching the coordinated multi-substation breaker-opening pattern used by Sandworm/Electrum's Industroyer2 against the Ukrainian grid in 2022.

| Rule ID | ta01a1b2-c3d4-4e5f-8071-001000000001 |
| --- | --- |
| Threat Actor | Sandworm / APT44 (Electrum) |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0855, attack.ics.t0831, attack.ics.t0879 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: IEC 104 Mass Coordinated Breaker-Open Command Burst id: ta01a1b2-c3d4-4e5f-8071-001000000001 status: experimental description: Detects multiple IEC 60870-5-104 Single/Double Command (open) operations issued to several distinct substation RTUs within a short time window from the same or correlated source(s), matching the coordinated multi-substation breaker-opening pattern used by Sandworm/Electrum's Industroyer2 against the Ukrainian grid in 2022. references: - MITRE ATT&CK Software S1072 (Industroyer2) - APT44 Unearthing Sandworm (Mandiant) threat_actor: Sandworm / APT44 (Electrum) tags: - attack.ics.t0855 - attack.ics.t0831 - attack.ics.t0879 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: iec104 detection: selection: proto: iec104 type_id: - 'C_SC_NA_1' - 'C_DC_NA_1' command_state: 'open' timeframe: 2m condition: selection ¦ count(distinct dst_ip) by src_ip > 3 falsepositives: - Legitimate wide-area protection scheme tripping multiple breakers during a genuine grid fault level: critical

## Modbus Holding Register Manipulation at Abnormal Cadence

Detects repeated Modbus holding register writes (function code 06/16) to the same device at a sustained, machine-paced cadence inconsistent with normal HMI/operator-driven control, resembling the FrostyGoop pattern of directly manipulating device parameters (e.g., heating system setpoints) via Modbus TCP for sabotage rather than reconnaissance.

| Rule ID | ta01a1b2-c3d4-4e5f-8071-001000000002 |
| --- | --- |
| Threat Actor | Sandworm / APT44 (assessed) |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0855, attack.ics.t0836 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Modbus Holding Register Manipulation at Abnormal Cadence id: ta01a1b2-c3d4-4e5f-8071-001000000002 status: experimental description: Detects repeated Modbus holding register writes (function code 06/16) to the same device at a sustained, machine-paced cadence inconsistent with normal HMI/operator-driven control, resembling the FrostyGoop pattern of directly manipulating device parameters (e.g., heating system setpoints) via Modbus TCP for sabotage rather than reconnaissance. references: - Dragos FrostyGoop malware analysis threat_actor: Sandworm / APT44 (assessed) tags: - attack.ics.t0855 - attack.ics.t0836 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: modbus detection: selection: proto: modbus function_code: - 6 - 16 timeframe: 5m condition: selection ¦ count() by src_ip, dst_ip, register_address > 30 falsepositives: - Automated control loop or PID controller performing legitimate high-frequency register writes level: high

## Mass Disk-Wiping Utility Execution Across Multiple Hosts

Detects near-simultaneous execution of disk-wiping indicators (vssadmin delete shadows, bcdedit boot-recovery disable, raw disk-sector overwrite utilities) across multiple hosts in a short window, matching the KillDisk/NotPetya pattern used by Sandworm to destroy IT systems adjacent to OT environments and disrupt recovery.

| Rule ID | ta01a1b2-c3d4-4e5f-8071-001000000003 |
| --- | --- |
| Threat Actor | Sandworm / APT44 |
| Status | experimental |
| MITRE ATT&CK | attack.t1490, attack.t1561, attack.ics.t0826 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: Mass Disk-Wiping Utility Execution Across Multiple Hosts id: ta01a1b2-c3d4-4e5f-8071-001000000003 status: experimental description: Detects near-simultaneous execution of disk-wiping indicators (vssadmin delete shadows, bcdedit boot-recovery disable, raw disk-sector overwrite utilities) across multiple hosts in a short window, matching the KillDisk/NotPetya pattern used by Sandworm to destroy IT systems adjacent to OT environments and disrupt recovery. references: - MITRE ATT&CK Software S0368 (NotPetya) - Dragos CRASHOVERRIDE/Industroyer analysis threat_actor: Sandworm / APT44 tags: - attack.t1490 - attack.t1561 - attack.ics.t0826 author: Mahesh date: 2026/07/01 logsource: category: process_creation product: windows detection: selection: CommandLine¦contains: - 'vssadmin delete shadows' - 'bcdedit /set' - 'wbadmin delete catalog' timeframe: 10m condition: selection ¦ count(distinct ComputerName) > 3 falsepositives: - Legitimate backup software performing scheduled shadow-copy cleanup across the fleet level: critical

## IEC 61850 GOOSE Unauthorized Publisher or Spoofed Message

Detects a GOOSE (Generic Object Oriented Substation Event) message on the substation LAN published by a source MAC/IED identity not present in the configured GOOSE Control Block (GoCB) subscriber/publisher list, or with a StNum/SqNum sequence anomaly, consistent with protection-relay spoofing techniques associated with the Industroyer malware's IEC 61850 module.

| Rule ID | ta01a1b2-c3d4-4e5f-8071-001000000004 |
| --- | --- |
| Threat Actor | Sandworm / APT44 (Electrum) |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0855, attack.ics.t0856 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: IEC 61850 GOOSE Unauthorized Publisher or Spoofed Message id: ta01a1b2-c3d4-4e5f-8071-001000000004 status: experimental description: Detects a GOOSE (Generic Object Oriented Substation Event) message on the substation LAN published by a source MAC/IED identity not present in the configured GOOSE Control Block (GoCB) subscriber/publisher list, or with a StNum/SqNum sequence anomaly, consistent with protection-relay spoofing techniques associated with the Industroyer malware's IEC 61850 module. references: - ESET Industroyer technical analysis threat_actor: Sandworm / APT44 (Electrum) tags: - attack.ics.t0855 - attack.ics.t0856 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: iec61850 detection: selection: proto: goose filter: publisher_mac¦in_list: 'configured_gocb_publishers' condition: selection and not filter falsepositives: - IED replaced or reconfigured without updating the GoCB allow-list level: critical

## Mass Lateral Deployment Tooling Preceding Suspected Wiper Activity

Detects PsExec, WMIC process-call-create, or Group Policy immediate-task deployment used to push the same executable to many hosts within minutes, the mass-propagation technique used by both BlackEnergy3 (2015 Ukraine grid attack) and NotPetya to stage destructive payloads enterprise-wide before pivoting toward OT-adjacent systems.

| Rule ID | ta01a1b2-c3d4-4e5f-8071-001000000005 |
| --- | --- |
| Threat Actor | Sandworm / APT44 |
| Status | experimental |
| MITRE ATT&CK | attack.t1570, attack.t1047, attack.ics.t0867 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Mass Lateral Deployment Tooling Preceding Suspected Wiper Activity id: ta01a1b2-c3d4-4e5f-8071-001000000005 status: experimental description: Detects PsExec, WMIC process-call-create, or Group Policy immediate-task deployment used to push the same executable to many hosts within minutes, the mass-propagation technique used by both BlackEnergy3 (2015 Ukraine grid attack) and NotPetya to stage destructive payloads enterprise-wide before pivoting toward OT-adjacent systems. references: - MITRE ATT&CK Software S0094 (BlackEnergy) - MITRE ATT&CK Software S0368 (NotPetya) threat_actor: Sandworm / APT44 tags: - attack.t1570 - attack.t1047 - attack.ics.t0867 author: Mahesh date: 2026/07/01 logsource: category: process_creation product: windows detection: selection: Image¦endswith: - '\PsExec.exe' - '\wmic.exe' CommandLine¦contains: 'process call create' timeframe: 5m condition: selection ¦ count(distinct TargetComputerName) by ParentImage, CommandLine > 10 falsepositives: - Authorized IT operations performing a scripted fleet-wide software deployment level: high

# APT34 / OilRig (Helix Kitten, Crambus, Hazel Sandstorm)

## DNS Tunneling Beacon From OT DMZ or Engineering Host

Detects DNS query patterns (high subdomain entropy, abnormal query volume to a single domain, or TXT/NULL record usage) from an OT DMZ or engineering host, consistent with the DNS-based C2 channels used by APT34/OilRig backdoors (e.g., Helminth, QUADAGENT) for covert command-and-control and data exfiltration.

| Rule ID | ta02a1b2-c3d4-4e5f-8071-002000000001 |
| --- | --- |
| Threat Actor | APT34 / OilRig (Helix Kitten) |
| Status | experimental |
| MITRE ATT&CK | attack.t1071.004, attack.t1568 |
| Severity | HIGH |

Sigma Rule (YAML):

title: DNS Tunneling Beacon From OT DMZ or Engineering Host id: ta02a1b2-c3d4-4e5f-8071-002000000001 status: experimental description: Detects DNS query patterns (high subdomain entropy, abnormal query volume to a single domain, or TXT/NULL record usage) from an OT DMZ or engineering host, consistent with the DNS-based C2 channels used by APT34/OilRig backdoors (e.g., Helminth, QUADAGENT) for covert command-and-control and data exfiltration. references: - MITRE ATT&CK Group G0049 (OilRig) threat_actor: APT34 / OilRig (Helix Kitten) tags: - attack.t1071.004 - attack.t1568 author: Mahesh date: 2026/07/01 logsource: category: dns product: generic detection: selection: src_zone: - OT_DMZ - engineering timeframe: 1h condition: selection ¦ count(distinct query_name) by src_ip, base_domain > 50 falsepositives: - Legitimate cloud-management agent using DNS-based polling for configuration updates level: high

## Web Shell Activity on Internet-Facing OT-Adjacent Server

Detects a newly created ASPX/PHP/JSP file in a web-accessible directory followed by an HTTP request to that file producing a child process spawn from the web server process, matching the web shell deployment technique repeatedly used by APT34/OilRig against Middle East government and energy-sector targets to gain a persistent internet-facing foothold near OT-adjacent networks.

| Rule ID | ta02a1b2-c3d4-4e5f-8071-002000000002 |
| --- | --- |
| Threat Actor | APT34 / OilRig (Crambus) |
| Status | experimental |
| MITRE ATT&CK | attack.t1505.003, attack.t1190 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: Web Shell Activity on Internet-Facing OT-Adjacent Server id: ta02a1b2-c3d4-4e5f-8071-002000000002 status: experimental description: Detects a newly created ASPX/PHP/JSP file in a web-accessible directory followed by an HTTP request to that file producing a child process spawn from the web server process, matching the web shell deployment technique repeatedly used by APT34/OilRig against Middle East government and energy-sector targets to gain a persistent internet-facing foothold near OT-adjacent networks. references: - Symantec Crambus Middle East Government Campaign threat_actor: APT34 / OilRig (Crambus) tags: - attack.t1505.003 - attack.t1190 author: Mahesh date: 2026/07/01 logsource: category: process_creation product: windows detection: selection: ParentImage¦endswith: - '\w3wp.exe' - '\httpd.exe' - '\nginx.exe' Image¦endswith: - '\cmd.exe' - '\powershell.exe' condition: selection falsepositives: - Legitimate web application admin tooling that intentionally shells out to system utilities level: critical

## Credential Harvesting Tool Execution on Engineering or SCADA Host

Detects execution of Mimikatz, LaZagne, or equivalent credential-dumping tool signatures (LSASS access, SAM/SECURITY hive read) on an engineering workstation or SCADA server, a technique APT34/OilRig has consistently used to expand access from an initial IT foothold toward OT-adjacent systems.

| Rule ID | ta02a1b2-c3d4-4e5f-8071-002000000003 |
| --- | --- |
| Threat Actor | APT34 / OilRig |
| Status | experimental |
| MITRE ATT&CK | attack.t1003.001, attack.t1003.002, attack.ics.t0859 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: Credential Harvesting Tool Execution on Engineering or SCADA Host id: ta02a1b2-c3d4-4e5f-8071-002000000003 status: experimental description: Detects execution of Mimikatz, LaZagne, or equivalent credential-dumping tool signatures (LSASS access, SAM/SECURITY hive read) on an engineering workstation or SCADA server, a technique APT34/OilRig has consistently used to expand access from an initial IT foothold toward OT-adjacent systems. references: - Trellix Iranian Cyber Capability 2026 threat_actor: APT34 / OilRig tags: - attack.t1003.001 - attack.t1003.002 - attack.ics.t0859 author: Mahesh date: 2026/07/01 logsource: category: process_creation product: windows detection: selection: Image¦contains: - 'mimikatz' - 'lazagne' selection_lsass: TargetImage¦endswith: '\lsass.exe' GrantedAccess: '0x1010' ComputerName¦in_list: 'l2_l3_ot_servers' condition: (selection or selection_lsass) falsepositives: - Authorized red-team or penetration-testing engagement against the OT environment level: critical

## Low-and-Slow HTTPS Beaconing Indicative of OilRig-Class Backdoor C2

Detects a host establishing periodic, low-volume HTTPS connections to the same external destination at a highly regular interval over an extended period, consistent with the long-dwell, low-and-slow C2 beaconing behavior documented for APT34/OilRig implants (e.g., Outer Space, Juicy Mix, STEALHOOK).

| Rule ID | ta02a1b2-c3d4-4e5f-8071-002000000004 |
| --- | --- |
| Threat Actor | APT34 / OilRig |
| Status | experimental |
| MITRE ATT&CK | attack.t1071.001, attack.t1102 |
| Severity | MEDIUM |

Sigma Rule (YAML):

title: Low-and-Slow HTTPS Beaconing Indicative of OilRig-Class Backdoor C2 id: ta02a1b2-c3d4-4e5f-8071-002000000004 status: experimental description: Detects a host establishing periodic, low-volume HTTPS connections to the same external destination at a highly regular interval over an extended period, consistent with the long-dwell, low-and-slow C2 beaconing behavior documented for APT34/OilRig implants (e.g., Outer Space, Juicy Mix, STEALHOOK). references: - ESET OilRig Outer Space and Juicy Mix threat_actor: APT34 / OilRig tags: - attack.t1071.001 - attack.t1102 author: Mahesh date: 2026/07/01 logsource: category: network_connection product: generic detection: selection: dst_port: 443 src_zone: - IT - OT_DMZ condition: selection ¦ beacon_interval_stddev(dst_ip) by src_ip < 5s and count() by src_ip, dst_ip > 100 falsepositives: - Legitimate SaaS application or monitoring agent with fixed-interval polling level: medium

# APT33 / Elfin (Refined Kitten)

## Shamoon-Style MBR Wiper Execution

Detects direct raw access to the physical disk device (\\.\PhysicalDrive0) combined with a service-creation event immediately followed by a system reboot, matching the Shamoon V3 wiper behavior attributed to APT33 in destructive attacks against Saudi Arabian and UAE energy-sector organizations.

| Rule ID | ta03a1b2-c3d4-4e5f-8071-003000000001 |
| --- | --- |
| Threat Actor | APT33 / Elfin (Refined Kitten) |
| Status | experimental |
| MITRE ATT&CK | attack.t1561.002, attack.t1529, attack.ics.t0826 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: Shamoon-Style MBR Wiper Execution id: ta03a1b2-c3d4-4e5f-8071-003000000001 status: experimental description: Detects direct raw access to the physical disk device (\\.\PhysicalDrive0) combined with a service-creation event immediately followed by a system reboot, matching the Shamoon V3 wiper behavior attributed to APT33 in destructive attacks against Saudi Arabian and UAE energy-sector organizations. references: - SOC Prime APT33 Shamoon and Filerase Wipers - MITRE ATT&CK Software S0140 (Shamoon) threat_actor: APT33 / Elfin (Refined Kitten) tags: - attack.t1561.002 - attack.t1529 - attack.ics.t0826 author: Mahesh date: 2026/07/01 logsource: category: process_creation product: windows detection: selection: CommandLine¦contains: '\\.\PhysicalDrive' condition: selection falsepositives: - Authorized low-level disk imaging/forensics tool used by IT operations level: critical

## Password Spray Against OT Remote Access Portal

Detects a single source IP attempting authentication against many distinct user accounts on the OT remote-access VPN or jump-server portal within a short window, the initial-access technique APT33 has consistently used (alongside supply-chain compromise) against aerospace, energy, and government targets.

| Rule ID | ta03a1b2-c3d4-4e5f-8071-003000000002 |
| --- | --- |
| Threat Actor | APT33 / Elfin |
| Status | experimental |
| MITRE ATT&CK | attack.t1110.003, attack.t1133 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Password Spray Against OT Remote Access Portal id: ta03a1b2-c3d4-4e5f-8071-003000000002 status: experimental description: Detects a single source IP attempting authentication against many distinct user accounts on the OT remote-access VPN or jump-server portal within a short window, the initial-access technique APT33 has consistently used (alongside supply-chain compromise) against aerospace, energy, and government targets. references: - Nozomi Networks Iranian APT Activity Middle East 2026 threat_actor: APT33 / Elfin tags: - attack.t1110.003 - attack.t1133 author: Mahesh date: 2026/07/01 logsource: category: authentication product: vpn detection: selection: auth_result: failure timeframe: 10m condition: selection ¦ count(distinct username) by src_ip > 15 falsepositives: - Misconfigured application performing repeated automated login attempts with stale credentials level: high

## Mass Malware Spreader Tool Execution Across Multiple Hosts

Detects a spreader-style executable enumerating network shares/hosts and then invoking PsExec against each in rapid succession, matching the OCLC.exe/Spreader.exe/SpreaderPSexec.exe toolchain APT33 used to distribute Shamoon V3 and Filerase wipers across victim networks.

| Rule ID | ta03a1b2-c3d4-4e5f-8071-003000000003 |
| --- | --- |
| Threat Actor | APT33 / Elfin |
| Status | experimental |
| MITRE ATT&CK | attack.t1570, attack.t1021.002 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: Mass Malware Spreader Tool Execution Across Multiple Hosts id: ta03a1b2-c3d4-4e5f-8071-003000000003 status: experimental description: Detects a spreader-style executable enumerating network shares/hosts and then invoking PsExec against each in rapid succession, matching the OCLC.exe/Spreader.exe/SpreaderPSexec.exe toolchain APT33 used to distribute Shamoon V3 and Filerase wipers across victim networks. references: - SOC Prime APT33 Shamoon and Filerase Wipers threat_actor: APT33 / Elfin tags: - attack.t1570 - attack.t1021.002 author: Mahesh date: 2026/07/01 logsource: category: process_creation product: windows detection: selection: CommandLine¦contains: - 'spreader' - 'PsExec.exe -d -accepteula' timeframe: 10m condition: selection ¦ count(distinct TargetComputerName) by ParentImage > 5 falsepositives: - Legitimate enterprise software deployment tool with similarly named binaries level: critical

# CHERNOVITE (PIPEDREAM / INCONTROLLER)

## OPC UA Server Enumeration Followed by Node Value Tampering

Detects an OPC UA Browse/Read sequence against a server immediately followed by a Write call from the same client within a short window, matching the CHERNOVITE/PIPEDREAM TAGRUN module's pattern of scanning an OPC UA server, reading its node structure, and then tampering with reported values to mask a subsequent manipulation.

| Rule ID | ta04a1b2-c3d4-4e5f-8071-004000000001 |
| --- | --- |
| Threat Actor | CHERNOVITE (PIPEDREAM / INCONTROLLER) |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0846, attack.ics.t0856 |
| Severity | HIGH |

Sigma Rule (YAML):

title: OPC UA Server Enumeration Followed by Node Value Tampering id: ta04a1b2-c3d4-4e5f-8071-004000000001 status: experimental description: Detects an OPC UA Browse/Read sequence against a server immediately followed by a Write call from the same client within a short window, matching the CHERNOVITE/PIPEDREAM TAGRUN module's pattern of scanning an OPC UA server, reading its node structure, and then tampering with reported values to mask a subsequent manipulation. references: - CISA AA22-103A PIPEDREAM/INCONTROLLER Advisory - TXOne PIPEDREAM Malware Analysis threat_actor: CHERNOVITE (PIPEDREAM / INCONTROLLER) tags: - attack.ics.t0846 - attack.ics.t0856 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: opcua detection: selection_browse: proto: opcua service: - Browse - Read selection_write: proto: opcua service: Write condition: selection_browse ¦ followed_by(selection_write, field=src_ip, window=2m) falsepositives: - Legitimate engineering tool performing a read-validate-write configuration sequence level: high

## CODESYS Protocol Command to Schneider Controller From Non-Engineering Host

Detects a CODESYS runtime protocol command (file delete, login brute-force attempt, file upload/download) directed at a Schneider Electric Modicon controller from a source host outside the designated engineering workstation list, matching the CHERNOVITE/PIPEDREAM CODECALL module's capability set.

| Rule ID | ta04a1b2-c3d4-4e5f-8071-004000000002 |
| --- | --- |
| Threat Actor | CHERNOVITE (PIPEDREAM / INCONTROLLER) |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0859, attack.ics.t0843, attack.ics.t0814 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: CODESYS Protocol Command to Schneider Controller From Non-Engineering Host id: ta04a1b2-c3d4-4e5f-8071-004000000002 status: experimental description: Detects a CODESYS runtime protocol command (file delete, login brute-force attempt, file upload/download) directed at a Schneider Electric Modicon controller from a source host outside the designated engineering workstation list, matching the CHERNOVITE/PIPEDREAM CODECALL module's capability set. references: - CISA AA22-103A PIPEDREAM/INCONTROLLER Advisory threat_actor: CHERNOVITE (PIPEDREAM / INCONTROLLER) tags: - attack.ics.t0859 - attack.ics.t0843 - attack.ics.t0814 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: codesys detection: selection: proto: codesys command: - 'file_delete' - 'login_attempt' - 'file_download' filter: src_ip¦in_list: 'designated_engineering_workstations' condition: selection and not filter falsepositives: - Backup engineering workstation used during primary host outage level: critical

## OMRON FINS/HTTP Backdoor Command Execution

Detects FINS protocol commands or HTTP requests to an OMRON controller's diagnostic/backdoor interface from a source not in the engineering allow-list, matching the CHERNOVITE/PIPEDREAM OMSHELL module's technique for scanning, connecting to, and issuing arbitrary commands against OMRON Sysmac NEX controllers.

| Rule ID | ta04a1b2-c3d4-4e5f-8071-004000000003 |
| --- | --- |
| Threat Actor | CHERNOVITE (PIPEDREAM / INCONTROLLER) |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0859, attack.ics.t0883 |
| Severity | HIGH |

Sigma Rule (YAML):

title: OMRON FINS/HTTP Backdoor Command Execution id: ta04a1b2-c3d4-4e5f-8071-004000000003 status: experimental description: Detects FINS protocol commands or HTTP requests to an OMRON controller's diagnostic/backdoor interface from a source not in the engineering allow-list, matching the CHERNOVITE/PIPEDREAM OMSHELL module's technique for scanning, connecting to, and issuing arbitrary commands against OMRON Sysmac NEX controllers. references: - CISA AA22-103A PIPEDREAM/INCONTROLLER Advisory threat_actor: CHERNOVITE (PIPEDREAM / INCONTROLLER) tags: - attack.ics.t0859 - attack.ics.t0883 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: fins detection: selection: proto: fins filter: src_ip¦in_list: 'designated_engineering_workstations' condition: selection and not filter falsepositives: - Newly deployed HMI/SCADA polling node not yet added to the allow-list level: high

## ASRock RGB Driver Exploitation Attempt (CVE-2020-15368)

Detects loading of the vulnerable ASRock RGB driver (AsrDrv103.sys or related) on an engineering workstation or SCADA server, the kernel-level privilege-escalation and Windows Defender-bypass vector incorporated into the CHERNOVITE/PIPEDREAM toolkit (CVE-2020-15368).

| Rule ID | ta04a1b2-c3d4-4e5f-8071-004000000004 |
| --- | --- |
| Threat Actor | CHERNOVITE (PIPEDREAM / INCONTROLLER) |
| Status | experimental |
| MITRE ATT&CK | attack.t1068, attack.t1562.001 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: ASRock RGB Driver Exploitation Attempt (CVE-2020-15368) id: ta04a1b2-c3d4-4e5f-8071-004000000004 status: experimental description: Detects loading of the vulnerable ASRock RGB driver (AsrDrv103.sys or related) on an engineering workstation or SCADA server, the kernel-level privilege-escalation and Windows Defender-bypass vector incorporated into the CHERNOVITE/PIPEDREAM toolkit (CVE-2020-15368). references: - CISA AA22-103A PIPEDREAM/INCONTROLLER Advisory threat_actor: CHERNOVITE (PIPEDREAM / INCONTROLLER) tags: - attack.t1068 - attack.t1562.001 author: Mahesh date: 2026/07/01 logsource: category: driver_load product: windows detection: selection: ImageLoaded¦contains: 'AsrDrv' ComputerName¦in_list: 'l2_l3_ot_servers' condition: selection falsepositives: - Legitimate ASRock motherboard RGB lighting utility installed on a non-OT workstation by mistake level: critical

# XENOTIME (TRITON / TRISIS)

## Triconex Safety Controller Engineering Session From Unauthorized Host

Detects a TriStation protocol engineering session (program download, key-switch state change, or memory write) to a Schneider Electric Triconex Safety Instrumented System controller from a host other than the single designated SIS engineering workstation, the exact access pattern used in the 2017 TRITON/TRISIS attack on a Saudi petrochemical facility.

| Rule ID | ta05a1b2-c3d4-4e5f-8071-005000000001 |
| --- | --- |
| Threat Actor | XENOTIME (TRITON / TRISIS) |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0859, attack.ics.t0800, attack.ics.t0837 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: Triconex Safety Controller Engineering Session From Unauthorized Host id: ta05a1b2-c3d4-4e5f-8071-005000000001 status: experimental description: Detects a TriStation protocol engineering session (program download, key-switch state change, or memory write) to a Schneider Electric Triconex Safety Instrumented System controller from a host other than the single designated SIS engineering workstation, the exact access pattern used in the 2017 TRITON/TRISIS attack on a Saudi petrochemical facility. references: - FireEye TRITON Attack on Safety Instrumented System - MITRE ATT&CK Group G0102 (XENOTIME) threat_actor: XENOTIME (TRITON / TRISIS) tags: - attack.ics.t0859 - attack.ics.t0800 - attack.ics.t0837 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: tristation detection: selection: proto: tristation filter: src_ip¦in_list: 'designated_sis_engineering_workstation' condition: selection and not filter falsepositives: - Approved SIS proof-test or logic-verification activity performed from a temporary certified engineering laptop level: critical

## Safety Controller Key-Switch or Mode Change Outside Proof-Test Window

Detects a Safety Instrumented System controller mode change (RUN to PROGRAM) occurring outside the scheduled, documented proof-test or maintenance window, a precondition exploited in the TRITON/TRISIS attack where the SIS controller was placed in PROGRAM mode to allow malicious logic upload.

| Rule ID | ta05a1b2-c3d4-4e5f-8071-005000000002 |
| --- | --- |
| Threat Actor | XENOTIME (TRITON / TRISIS) |
| Status | experimental |
| MITRE ATT&CK | attack.ics.t0858, attack.ics.t0800 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: Safety Controller Key-Switch or Mode Change Outside Proof-Test Window id: ta05a1b2-c3d4-4e5f-8071-005000000002 status: experimental description: Detects a Safety Instrumented System controller mode change (RUN to PROGRAM) occurring outside the scheduled, documented proof-test or maintenance window, a precondition exploited in the TRITON/TRISIS attack where the SIS controller was placed in PROGRAM mode to allow malicious logic upload. references: - FireEye TRITON Attack on Safety Instrumented System threat_actor: XENOTIME (TRITON / TRISIS) tags: - attack.ics.t0858 - attack.ics.t0800 author: Mahesh date: 2026/07/01 logsource: category: ics-network product: tristation detection: selection: proto: tristation mode_change: 'PROGRAM' filter: change_ticket_active: true condition: selection and not filter falsepositives: - Authorized proof test with a change ticket not correctly tagged in the work-order system level: critical

# Generalized Historical Attack Patterns

## Engineering Workstation Reflective/Memory-Only Process Injection

Detects indicators of reflective DLL loading or memory-only code execution (no corresponding on-disk image, anomalous memory protection flags) on an engineering workstation, the evasion class of technique pioneered by Stuxnet's rootkit components to hide malicious PLC project-file modifications from the engineering software's normal view.

| Rule ID | ta06a1b2-c3d4-4e5f-8071-006000000001 |
| --- | --- |
| Threat Actor | Generalized (Stuxnet-class TTP) |
| Status | experimental |
| MITRE ATT&CK | attack.t1055, attack.ics.t0851 |
| Severity | HIGH |

Sigma Rule (YAML):

title: Engineering Workstation Reflective/Memory-Only Process Injection id: ta06a1b2-c3d4-4e5f-8071-006000000001 status: experimental description: Detects indicators of reflective DLL loading or memory-only code execution (no corresponding on-disk image, anomalous memory protection flags) on an engineering workstation, the evasion class of technique pioneered by Stuxnet's rootkit components to hide malicious PLC project-file modifications from the engineering software's normal view. references: - MITRE ATT&CK Software S0603 (Stuxnet) threat_actor: Generalized (Stuxnet-class TTP) tags: - attack.t1055 - attack.ics.t0851 author: Mahesh date: 2026/07/01 logsource: category: process_access product: windows detection: selection: ComputerName¦in_list: 'designated_engineering_workstations' CallTrace¦contains: 'UNKNOWN' condition: selection falsepositives: - Legitimate licensing/anti-tamper module used by some engineering software suites level: high

## Mass File Encryption or Rename Activity on OT Historian or Engineering Host

Detects a high rate of file rename/modify events with new or unusual extensions on an OT historian or engineering workstation within a short window, the ransomware-encryption pattern that forced precautionary OT shutdowns in the Colonial Pipeline (DarkSide) and Norsk Hydro (LockerGoga) incidents even though the ransomware itself did not directly target OT systems.

| Rule ID | ta06a1b2-c3d4-4e5f-8071-006000000002 |
| --- | --- |
| Threat Actor | Generalized (Ransomware precursor to OT shutdown) |
| Status | experimental |
| MITRE ATT&CK | attack.t1486, attack.ics.t0826 |
| Severity | CRITICAL |

Sigma Rule (YAML):

title: Mass File Encryption or Rename Activity on OT Historian or Engineering Host id: ta06a1b2-c3d4-4e5f-8071-006000000002 status: experimental description: Detects a high rate of file rename/modify events with new or unusual extensions on an OT historian or engineering workstation within a short window, the ransomware-encryption pattern that forced precautionary OT shutdowns in the Colonial Pipeline (DarkSide) and Norsk Hydro (LockerGoga) incidents even though the ransomware itself did not directly target OT systems. references: - CISA Colonial Pipeline Ransomware Advisory - Norsk Hydro LockerGoga Incident Reports threat_actor: Generalized (Ransomware precursor to OT shutdown) tags: - attack.t1486 - attack.ics.t0826 author: Mahesh date: 2026/07/01 logsource: category: file_event product: windows detection: selection: ComputerName¦in_list: - 'l3_scada_servers' - 'designated_engineering_workstations' EventType: 'rename' timeframe: 2m condition: selection ¦ count() by ComputerName > 50 falsepositives: - Legitimate bulk file-management or archival operation performed by IT level: critical

# Deployment & Tuning Notes

- Threat-intel correlation: feed the threat_actor metadata field into your TIP/SIEM case management so a triggered alert auto-suggests the relevant actor profile and historical incident pattern to the analyst during triage.

- Several rules depend on protocol parsers not present in every OT monitoring platform (CODESYS, FINS, TriStation, IEC 61850 GOOSE) — verify Nozomi Guardian/Vantage or your Zeek deployment has the corresponding dissector enabled before activating these.

- Safety-system rules (XENOTIME/TRITON group) should be reviewed jointly with process safety engineering; these are intentionally tuned toward zero tolerance given the physical-harm potential demonstrated by the 2017 incident.

- Sandworm-pattern rules combining IT wiper indicators with OT protocol bursts are strongest when correlated together — consider a composite/meta-detection that raises severity when both an IT-side mass-wiper indicator and an OT-side coordinated command burst fire within the same incident window.

- CHERNOVITE/PIPEDREAM rules are pre-positioned defenses against a toolkit that, as of this writing, has not been confirmed used in a live attack — keep at “high” rather than “critical” severity unless corroborated by other indicators, to avoid alert fatigue.

- Revisit and refresh this library periodically: threat-actor TTPs evolve (APT34 alone has shifted tooling multiple times since 2017), and new groups with OT intent and capability continue to emerge.